// -*- text -*-

/**

* \file Geographic.doc

* \brief Documentation for GeographicLib

*

* Written by Charles Karney <charles@karney.com> and licensed under the

* LGPL.  For more information, see http://geographiclib.sourceforge.net/

**********************************************************************/

// $Id: Geographic.doc 6829 2010-05-21 15:12:33Z karney $

/**

\mainpage Geographic library

\author <a href="http://charles.karney.info/">Charles Karney</a>

<charles@karney.com>

\version 1.2

\date 2010-05-21

\section abstract Abstract

%GeographicLib is a small set of <a href="annotated.html">C++

classes</a> for performing conversions between geographic, UTM, UPS,

MGRS, geocentric, and local cartesian coordinates, for geoid height

calculations, and for solving geodesic problems.  The emphasis is on

returning accurate results with errors close to round-off (about

5&ndash;15 nm).  In addition, various properties of the \ref

transversemercator are described and an accurate algorithm for \ref

geodesic is given.

This library is <i>not</i> a general purpose projection library; use

<a href="http://trac.osgeo.org/proj/">proj.4</a> for that.  On the other

hand, it does provide the core functionality offered by

<a href="http://earth-info.nima.mil/GandG/geotrans/">geotrans</a> and it

cures several of the defects of that package.

\section download Download

The main project page is at

-  <a href="http://sourceforge.net/projects/geographiclib">

http://sourceforge.net/projects/geographiclib	</a>

.

The code is available for download at

- <a href="http://sourceforge.net/projects/geographiclib/files/distrib/geographiclib-1.2.tar.gz/download">

geographiclib-1.2.tar.gz</a>

- <a href="http://sourceforge.net/projects/geographiclib/files/distrib/geographiclib-1.2.zip/download">

geographiclib-1.2.zip</a>

.

as either a compressed tar file (tar.gz) or a zip file.  (The two archives

have identical contents.)

It is licensed under the

<a href="http://www.gnu.org/licenses/lgpl-3.0-standalone.html">LGPL</a>.

For more information, see http://geographiclib.sourceforge.net/

\section contents Contents

- \ref intro

- \ref start

- \ref organization

- \ref transversemercator

- \ref tmvarious

- \ref extend

- \ref testmerc

- \ref tmerrors

- \ref tmseries

- \ref geodesic

- \ref geoddesc

- \ref testgeod

- \ref geoderrors

- \ref geodseries

- \ref geodmisc

- \ref geocentric

- \ref geoid

- \ref geoidinst

- \ref geoidformat

- \ref geoidinterp

- \ref geoidcache

- \ref testgeoid

- \ref utilities

- \ref geoconvert

- \ref geod

- \ref transversemercatortest

- \ref cartconvert

- \ref equidistanttest

- \ref geoideval

- \ref old

<center>

Forward to \ref intro.

</center>

**********************************************************************/

/**

\page intro Introduction

<center>

Forward to \ref organization.  Up to \ref contents.

</center>

%GeographicLib offers a C++ interfaces to a small (but important!) set

of geographic transformations.  It grew out of a desire to improve on

the <a href="http://earth-info.nima.mil/GandG/geotrans/">geotrans</a>

package for transforming between geographic and MGRS coordinates.  At

present, %GeographicLib provides UTM, UPS, MGRS, geocentric, and local

cartesian projections, geoid heights, and classes for geodesic

calculations.

The goals of %GeographicLib are:

- Accuracy.  In most applications the accuracy is close to round-off,

about 5 nm.  Even though in many geographic applications 1 cm is

considered "accurate enough", there is little penalty in providing

much better accuracy.  If situations where a faster approximate

algorithm is necessary, %GeographicLib offers an accurate benchmark to

guide the development.

- Completeness.  For each of the projections included, an attempt is

made to provide a complete solution.  For example,

GeographicLib::Geodesic::Inverse works for anti-podal points.

Similarly, GeographicLib::Geocentric.Reverse will return accurate

geodetic coordinates even for points close to the center of the

earth.

- C++ interface.  For the projection methods, this allows encapsulation

of the ellipsoid parameters.

- Emphasis on projections necessary for analysing military data.

- Uniform treatment of UTM/UPS.  The GeographicLib::UTMUPS class treats

UPS as zone 0.  This simplifies conversions between UTM and UPS

coordinates, etc.

- Well defined and stable conventions for the conversion between

UTM/UPS to MGRS coordinates.

- Detailed internal documentation on the algorithms.  For the most part

%GeographicLib uses published algorithms and references are given.  If

changes have been made (usually to improve the numerical accuracy),

these are described in the code.

Various \ref utilities are provided with the library.  These illustrate

the use of the library and are useful in their own right.  This library

and the utilities have been tested with g++ 4.3 under Linux and with MS

Visual Studio 2005

The section \ref transversemercator documents various properties of this

projection.

The section \ref geodesic documents (still rather incompletely) the

method of solving the geodesic problem.

Undoubtedly, bugs lurk in this code and in the documentation.  Please

report any you find to <charles@karney.com>.

<center>

Forward to \ref start.  Up to \ref contents.

</center>

**********************************************************************/

/**

\page start Getting Started

<center>

Back to \ref intro.  Forward to \ref organization.  Up to \ref contents.

</center>

The following steps will get you started with %GeographicLib:

- Download the

<a href="http://sourceforge.net/projects/geographiclib/files/distrib/geographiclib-1.2.tar.gz/download">

source</a>.

- Unpack the tar file somewhere convenient with, e.g.,

- tar xfpvz geographiclib-1.2.tar.gz

.

This will create a geographiclib-1.2 directory.  (Alternatively,

download and unpack the zip file.)

- Change to the geographiclib-1.2 directory

- cd geographiclib-1.2

- Create the library and the utility programs

- make all

.

The compiled utility programs are in the tools directory.

- windows/GeographicLib-vc9.sln (resp. vc8) is MS Visual Studio 2008

(resp. 2005) solution file which compiles the library and the utility

programs under Windows.

- Try out \ref geoconvert (for help, type cd tools; ./GeoConvert -h)

- echo 33.3 44.4 | ./GeoConvert -m -p -3

- echo 38SMB4484 | ./GeoConvert -d

- Similarly, try out \ref geod, \ref transversemercatortest,

\ref cartconvert, and \ref equidistanttest.

- Read the section, \ref organization, for an overview of the library.

- Browse the <a href="annotated.html">Class List</a> for full documentation

on the library.

- Read the section, \ref transversemercator, for more information on

this projection.

- If you use the library frequently, install it, e.g., to \$(PREFIX) =

/usr/local with

- make PREFIX=/usr/local install install-doc

Here's a list of some of the abbreviations used here with links to the

corresponding Wikipedia articles:

- <a href="http://en.wikipedia.org/wiki/WGS84">

WGS84</a>, World Geodetic System 1984.

- <a href="http://en.wikipedia.org/wiki/Universal_Transverse_Mercator_coordinate_system">

UTM</a>, Universal Transverse Mercator coordinate system.

- <a href="http://en.wikipedia.org/wiki/Universal_Polar_Stereographic">

UPS</a>, Universal Polar Stereographic coordinate system.

- <a href="http://en.wikipedia.org/wiki/Military_grid_reference_system">

MGRS</a>, Military Grid Reference System.

- <a href="http://en.wikipedia.org/wiki/Geoid">

EGM</a>, Earth Gravity Model.

<center>

Back to \ref intro.  Forward to \ref organization.  Up to \ref contents.

</center>

**********************************************************************/

/**

\page organization Code organization

<center>

Back to \ref start.  Forward to \ref transversemercator.  Up to \ref contents.

</center>

Here is a brief description of the relationship between the various

components of %GeographicLib.

GeographicLib::TransverseMercator, GeographicLib::PolarStereographic,

and GeographicLib::LambertConformalConic provide the basic projections.

The constructors for these classes specify the ellipsoid and the forward

and reverse projections are implemented as const member functions.

TransverseMercator uses Kr&uuml;ger's series which have been extended to

sixth order in the square of the eccentricity.  PolarStereographic and

LambertConformalConic use the exact formulas for the projections (from

Snyder).

GeographicLib::TransverseMercator::UTM and

GeographicLib::PolarStereographic::UPS are const static instantiations

specific for the WGS84 ellipsoid with the UTM and UPS scale factors.

(These do \e not add the standard false eastings or false northings for

UTM and UPS.)  Similarly GeographicLib::LambertConformalConic::Mercator

is a const static instantiation of this projection for a WGS84 ellipsoid

and a standard parallel of 0 (which gives the Mercator projection).

GeographicLib::UTMUPS uses TransverseMercator::UTM and

PolarStereographic::UPS to perform the UTM and UPS

projections.  The class offers a uniform interface to UTM and UPS by

treating UPS as UTM zone 0.  This class stores no internal state and the

forward and reverse projections are provided via static member

functions.  The forward projection offers the ability to override the

standard UTM/UPS choice and the UTM zone.

GeographicLib::MGRS transforms between UTM/UPS coordinates and MGRS.

UPS coordinates are handled as UTM zone 0.  This class stores no

internal state and the forward (UTM/UPS to MGRS) and reverse (MGRS to

UTM/UPS) conversions are provided via static member functions.

GeographicLib::GeoCoords holds a single geographic location which may be

specified as latitude and longitude, UTM or UPS, or MGRS.  Member

functions are provided to convert between coordinate systems and to

provide formatted representations of them.

\ref geoconvert is a simple command line utility to provide access to

the GeoCoords class.

GeographicLib::TransverseMercatorExact is a drop in replacement for

TransverseMercator which uses the exact formulas, based on elliptic

functions, for the projection as given by Lee.  \ref

transversemercatortest is a simple command line utility to test to the

TransverseMercator and TransverseMercatorExact.

GeographicLib::Geodesic and GeographicLib::GeodesicLine perform geodesic

calculations.  The constructor for GeographicLib::Geodesic specifies the

ellipsoid and the direct and inverse calculations are implemented as

const member functions.  GeographicLib::Geocentric::WGS84 is a const

static instantiation of Geodesic specific for the WGS84 ellipsoid.  In

order to perform a series of direct geodesic calculations on a single

line, the GeographicLib::GeodesicLine class can be used.  This packages

all the information needed to specify a geodesic.  A const member

function returns the coordinates a specified distance from the starting

point.  \ref geod is a simple command line utility to perform geodesic

calculations.  GeographicLib::AzimuthalEquidistant computes the

azimuthal equidistant projection using Geodesic.

GeographicLib::CassiniSoldner computes the Cassini-Soldner projection

which is a cylindrical equidistant projection using Geodesic. \ref

equidistanttest is a simple command line utility to exercise these two

projections.

GeographicLib::Geocentric and GeographicLib::LocalCartesian convert between

geodetic and geocentric or a local cartesian system.  The constructor for

specifies the ellipsoid and the forward and reverse projections are

implemented as const member functions.  GeographicLib::Geocentric::WGS84 is a

const static instantiation of Geocentric specific for the WGS84 ellipsoid.

\ref cartconvert is a simple command line utility to provide access to

these classes.

GeographicLib::Geoid evaluates geoid heights by interpolation.  This is

provided by the operator() member function.  \ref geoideval is a simple

command line utility to provide access to this class.  This class

requires installation of data files for the various geoid models; see

\ref geoidinst for details.

GeographicLib::Constants, GeographicLib::Math, GeographicLib::DMS, and

GeographicLib::EllipticFunction are utility classes which are used

internally by the library.  One important definition is

GeographicLib::Math::real_t which is the type used for real numbers.

This allows the library to be easily switched to using floats, doubles,

or long doubles.  However all the testing has been with real_t set to

double and the library should be installed in this way.

In order to avoid name clashes, all there classes are in the

GeographicLib namespace.  The header files are included with the

GeographicLib/ directory prefix.  If compiling within the

%GeographicLib/src or %GeographicLib/tools directories, "../include"

should be included in the header search path (e.g., with -I../include).

The cpp files implementing the classes include "using namespace std;",

however, the headers files and the utility programs do not.

There's an "install" target in the Makefile which installs the library

components into \$(PREFIX)/lib, \$(PREFIX)/include/GeographicLib,

\$(PREFIX)/bin, \$(PREFIX)/share/GeographicLib/doc.  Once installed, the

library can be used by setting

- INCLUDEPATH = \$(PREFIX)/include

- LIBPATH = \$(PREFIX)/lib

.

in tools/Makefile.

The low level projection classes, e.g., TransverseMercator and

PolarStereographic, do not throw exceptions.  Most of the other classes,

e.g., UTMUPS, MGRS, and GeoCoords all check their arguments and throw

std::out_of_range exceptions with a explanatory message.  See

GeoConvert.cpp for an example of how exceptions can be handled.  Geoid

is exceptional in than file errors can cause exceptions to be thrown

when evaluating the geoid at a valid location.

<center>

Back to \ref start.  Forward to \ref transversemercator.  Up to \ref contents.

</center>

**********************************************************************/

/**

\page transversemercator Transverse Mercator Projection

<center>

Back to \ref organization.  Forward to \ref geodesic.  Up to \ref contents.

</center>

\b NOTE 1: Some of the material in this section has been included in

- Charles F. F. Karney,

<a href="http://arxiv.org/abs/1002.1417">

Transverse Mercator with an accuracy of a few nanometers,</a>

Feb. 2010,

<a href="http://arxiv.org/abs/1002.1417">arXiv:1002.1417</a>.

\b NOTE 2: The figures in this section are relatively small in order to

allow them to be displayed quickly.  The "Fig. N" link beneath each

figure points to a PDF version of the figure.  These are in vector

format and allow the figures to be magnified to show the details more

clearly.  The numbering of the figures matches that in the paper cited

above.

The primary sources for the material presented here are:

- J. P. Snyder,

<a href="http://pubs.er.usgs.gov/usgspubs/pp/pp1395">Map Projections: A

Working Manual</a>,

USGS Professional Paper 1395 (1987), pp. 48&ndash;66.

This contains an overview of the transverse Mercator projection (and

many other projections). However, the approximations given for

this projection are rather crude.  The terminology for the various

transverse Mercator map projections is taken form J. P. Snyder,

Flattening the Earth (Chicago Univ. Press, 1993).

- L. Kr&uuml;ger, <a

href="http://dx.doi.org/10.2312/GFZ.b103-krueger28"> Konforme

Abbildung des Erdellipsoids in der Ebene</a> (Conformal mapping of

the ellipsoidal earth to the plane), Royal Prussian Geodetic Institute,

New Series 52, 172 pp. (1912).

The derives the series solution for the transverse Mercator

projection of an ellipsoid.  This gives very good accuracy and this

method or an extension of it to higher-order provides the most accurate

algorithms for the projection valid in some zone near the central

meridian.  It is also of interest to see the systematic use of

logarithms in the pre-electronic and pre-mechanical age (when

computers were human).

- L. P. Lee,

Conformal Projections Based on Elliptic Functions,

(B. V. Gutsell, Toronto, 1976), 128pp.,

ISBN: 0919870163

(Also appeared as:

Monograph 16, Suppl. No. 1 to Canadian Cartographer, Vol 13).

Part V, pp. 67&ndash;101,

<a href="http://dx.doi.org/10.3138/X687-1574-4325-WM62">Conformal

Projections Based On Jacobian Elliptic Functions</a>, gives closed

form expressions for the transverse Mercator projection based on work

by E. H. Thompson (1945).  This may be purchased for a modest sum.  Other

useful sections are Part I, pp. 1&ndash;3 and Part II, pp. 4&ndash;14.

.

\section tmvarious Survey of transverse Mercator projections

The following is adapted from Lee (1976) particularly pp. 92&ndash;101.

The terminology is from Snyder (1993) pp. 159&ndash;161.

The transverse Mercator projection is a conformal projection of the

ellipsoid in which the central meridian is straight.  Within these

constrains there are various projections that differ in how the scale

varies along the central meridian.  The simplest such projection from an

algorithmic point of view is the Gauss-Schreiber projection which

consists of a conformal projection from the ellipsoid to a sphere

followed by the spherical transverse Mercator projection.  This results

in a projection whose scale on the central meridian increases from the

equator to the pole (by an amount proportional to the flattening).  In

the Gauss-Kr&uuml;ger projection, the scale along the central meridian

is held constant.  This projection is usually expressed as a series

which is valid only sufficiently close to the central meridian.  (This

is implemented by GeographicLib::TransverseMercator.)  The projection

can however be expressed in closed form in terms of the Thompson

transverse Mercator projection (where the scale along the central

meridian decreases from the equator to the pole by an amount

proportional to the flattening).  (This method of computing the

Gauss-Kr&uuml;ger projection is implemented by

GeographicLib::TransverseMercatorExact.)

The graticules (lines of constant latitude and longitude) for all these

projections are shown in Figs. 1(a&ndash;c).  These figures all use an

ellipsoid with eccentricity \e e = 1/10 (inverse flattening = 10 (3

sqrt(11) + 10) = 199.5, i.e., somewhat flatter than the WGS84 ellipsoid)

and the projections have all been scaled so that the distance from the

equator to the north pole is unity.  One eighth of the ellipsoid is

shown in these figures, 0<sup>o</sup> <= latitude <= 90<sup>o</sup> and

0<sup>o</sup> <= longitude <= 90<sup>o</sup>.  To obtain the graticule

for the entire ellipsoid, reflect these figures in \e x = 0, \e y = 0,

and \e y = 1.  The blue lines show latitude and longitude at multiples

of 10<sup>o</sup>.  The green lines show 1<sup>o</sup> intervals for

longitude in [80, 90] and latitude in [0, 10].  In all three figures,

longitude = 0<sup>o</sup> lies on \e x = 0 and longitude =

90<sup>o</sup> lies on \e y = 1.

<center>

<img src="gauss-schreiber-graticule-a.png"

width=667 height=305 alt="Gauss-Schreiber transverse Mercator graticule

(scaled)">

<a href="gauss-schreiber-graticule-a.pdf">Fig. 1(a)</a>

</center>\n

The graticule for the Gauss-Schreiber transverse Mercator projection.

The equator lies on \e y = 0.

Compare this with Lee, Fig. 1 (right), which shows the graticule for

half a sphere, but note that in his notation \e x and \e y have switched

meanings.  The graticule for the ellipsoid differs from that for a

sphere only in that the latitude lines have shifted slightly.  (The

conformal transformation from an ellipsoid to a sphere merely relabels

the lines of latitude.)  This projection places the point latitude =

0<sup>o</sup>, longitude = 90<sup>o</sup> at infinity.

<center>

<img src="gauss-krueger-graticule-a.png"

width=667 height=305 alt="Gauss-Krueger transverse Mercator graticule

(scaled)">

<a href="gauss-krueger-graticule-a.pdf">Fig. 1(b)</a>

</center>\n

The graticule for the Gauss-Kr&uuml;ger transverse Mercator projection.

The equator lies on \e y = 0 for longitude < 81<sup>o</sup>; beyond

this, it arcs up to meet \e y = 1.  Compare this with Lee, Fig. 45

(upper), which shows the graticule for the International Ellipsoid.  Lee,

Fig. 46, shows the graticule for the entire ellipsoid.  This projection

(like the Thompson projection) projects the ellipsoid to a finite area.

<center>

<img src="thompson-tm-graticule-a.png"

width=667 height=305 alt="Thompson transverse Mercator graticule

(scaled)">

<a href="thompson-tm-graticule-a.pdf">Fig. 1(c)</a>

</center>\n

The graticule for the Thompson transverse Mercator projection.  The

equator lies on \e y = 0 for longitude < 81<sup>o</sup>; at longitude =

81<sup>o</sup>, it turns by 120<sup>o</sup> and heads for \e y = 1.

Compare this with Lee, Fig. 43, which shows the graticule for the

International Ellipsoid.  Lee, Fig. 44, shows the graticule for the

entire ellipsoid.  This projection (like the Gauss-Kr&uuml;ger

projection) projects the ellipsoid to a finite area.

Of the three transverse Mercator projections given here the

Gauss-Kr&uuml;ger projection is by far the most important.  It is the

basis of the UTM system.  If "transverse Mercator projection" is used

without further elaboration, then, nearly always, it refers to

the Gauss-Kr&uuml;ger projection.

\section extend Properties far from the central meridian

The divergent behavior of the Gauss-Schreiber projection as latitude =

0<sup>o</sup>, longitude = 90<sup>o</sup> is approached is unsurprising

given the behavior of the standard Mercator projection at the poles.  On

the other hand, the lack of divergence in the Gauss-Kr&uuml;ger

projection (and similarly in the Thompson projection) is perhaps

puzzling.  We explore the behavior more carefully in this section.

Firstly, the Gauss-Kr&uuml;ger projection \e does have a singularity at

latitude = 0<sup>o</sup> and longitude = 90<sup>o</sup>(1 - \e e), where

\e e is the eccentricity of the ellipsoid.  The choice of \e e = 1/10

for Figs. 1&ndash;3 was made to ensure that one of the graticule lines

(longitude = 81<sup>o</sup>) covered this singularly.  The singularly is

relatively benign.  The Thompson projection contains a cube root

singularity with causes the corner in the equator.  The transformation

from Thompson to Gauss-Kr&uuml;ger "repairs" this to lowest order so

that there's no kink in the equation.  Nevertheless, a weaker

singularity remains (causing a discontinuous change in the radius of

curvature for the equator).  Thus Gauss-Kr&uuml;ger has a branch point

at latitude = 0<sup>o</sup> and longitude = 90<sup>o</sup>(1 - \e e) and

the projection is analytic provided this point is avoided.  However the

results obtained for the projection will depend on which sheet of the

projection is used or which route around the branch point is taken.

The "standard" convention for projecting a geographic position is to

follow the central meridian from the equator to the target latitude,

then to follow the line of constant latitude to the target longitude.

This is equivalent to placing two branch cuts on the equator in the

longitude ranges +/- [90<sup>o</sup>(1 - \e e), 90<sup>o</sup>(1 + \e

e)].  For points on equator for |longitude| > 90<sup>o</sup>(1 - \e e),

the path should go to the north of branch point.  This convention

corresponds to Fig. 1(b), above, after suitable reflections to cover the

ellipsoid and to Lee, Fig. 46.

In order to explore the behavior to the right of the equator in Fig. 1(b),

we define an "extended" domain for the projection, this consists of the

union of

- 0<sup>o</sup> <= latitude <= 90<sup>o</sup> and

0<sup>o</sup> <= longitude <= 90<sup>o</sup>

- -90<sup>o</sup> < latitude <= 0<sup>o</sup> and

90<sup>o</sup>(1 - \e e) <= longitude <= 90<sup>o</sup>

.

The rule for reaching the second area here is to move on the central

meridian to a small positive latitude, move at constant latitude to the

target longitude and then to move on a meridian to the target latitude.

In this case, the branch cut emanates in a south-westerly direction from

the branch point.  Following this prescription the range of the

projection now consists of the union of

- 0 <= \e x and

0 <= \e y/(\e k<sub>0</sub> \e a) <= E(\e e<sup>2</sup>)

- K(1 - \e e<sup>2</sup>) - E(1 - \e e<sup>2</sup>)

<= \e x/(\e k<sub>0</sub> \e a) and \e y <= 0

.

where \e k<sub>0</sub> is the central scale factor, \e a is the major

radius of the ellipsoid, K and E are complete elliptic integrals.

Setting the optional argument \e extendp = true in the constructor for

GeographicLib::TransverseMercatorExact provides access to this extended

domain.

Figures 4&ndash;5 illustrate the properties of the Gauss-Kr&uuml;ger

transverse Mercator projection in this extended domain.  These figures

use an ellipsoid with eccentricity 1/10 (as in Figs. 1&ndash;3) and with

\e a = 1/E(0.01) = 0.6382 and \e k<sub>0</sub> = 1.  The branch point in

this case lies at lat = 0<sup>o</sup>, lon = 81<sup>o</sup> or \e x =

(K(0.99) - E(0.99))/E(0.01) = 1.71, \e y = 0.

<center>

<img src="gauss-krueger-graticule.png"

width=673 height=323 alt="Gauss-Krueger transverse Mercator

graticule">\n

<a href="gauss-krueger-graticule.pdf">Fig. 3(a)</a>

</center>

Figure 4 shows the graticule the extended domain.  The blue lines show

latitude and longitude at multiples of 10<sup>o</sup>.  The green lines

show 1<sup>o</sup> intervals for longitude in [80, 90] and latitude in

[-5, 10].

<center>

<img src="gauss-krueger-convergence-scale.png"

width=673 height=323 alt="Gauss-Krueger transverse Mercator

convergence and scale">

<a href="gauss-krueger-convergence-scale.pdf">Fig. 3(b)</a>

</center>\n

Figure 5 shows the convergence and scale for the Gauss-Kr&uuml;ger

transverse Mercator projection in the extended domain.  The blue lines

emanating from the top left corner (the north pole) are lines of

constant convergence.  Convergence = 0<sup>o</sup> is given by the

dog-legged line joining the points (0,1), (0,0), (1.71,0), (1.71,-inf).

Convergence = 90<sup>o</sup> is given by the line y = 1.  The other

lines show multiples of 10<sup>o</sup> between 0<sup>o</sup> and

90<sup>o</sup>.  The other blue, the green and the black lines show

scale = 1 thru 2 at intervals of 0.1, 2 thru 15 at intervals of 1, and

15 thru 35 at intervals of 5.  Multiples of 5 are shown in black,

multiples of 1 are shown in blue, and the rest are shown in green.

Scale = 1 is given by the line segment (0,0) to (0,1).  The red line

shows the equator between lon = 81<sup>o</sup> and 90<sup>o</sup>.  The

scale and convergence at the branch point are 1/\e e = 10 and

0<sup>o</sup>, respectively.

Symmetries in the projection can now be employed to extend the

Gauss-Kr&uuml;ger transverse Mercator projection with arbitrary rules

for how to circumvent the branch point.  The symmetries are equivalent

to placing mirrors on the lines segments

- 0 <= \e x, \e y = 1

- \e x = 0, 0 <= \e y <= 1

- 0 <= \e x <= 1.71, \e y = 0

- \e x = 1.71, \e y <= 0

In order to understand the projection better, it is instructive to

examine the Thompson projection in the extended domain.

<center>

<img src="thompson-tm-graticule.png"

width=661 height=321 alt="Thompson transverse Mercator graticule">

<a href="thompson-tm-graticule.pdf">Fig. 3(c)</a>

</center>\n

Figure 6 shows the graticule for the Thompson transverse Mercator

projection for the extended domain.  The range of the projection is the

rectangular region shown

- 0 <= \e u <= K(\e e<sup>2</sup>),

0 <= \e v <= K(1 - \e e<sup>2</sup>)

.

The coloring of the lines is the same as Fig. 3(a), except that latitude

lines extended down to -10<sup>o</sup> and a red line has been added

showing the line \e y = 0 for \e x > 1.71 in the Gauss-Kr&uuml;ger

projection (Fig. 3(a)).  The extended Thompson projection figure has

reflection symmetry on all the four sides of Fig. 3(c).

In transforming from Thompson to Gauss-Kr&uuml;ger, the right angle at

lower right corner of Fig. 3(c) expands by 3 to 270<sup>o</sup> to produce

the outside corner at \e x = 1.71 and \e y = 0 in Fig. 3(a).  The top right

corner of Fig. 3(c) represents the south pole and this is transformed to

infinity in the extended Gauss-Kr&uuml;ger projection.  Despite the

apparent similarities, the behaviors of the extended Thompson projection

near the north and south poles (top left and top right corners in

Fig. 3(c)) are rather different.  The singularity for the north pole is

merely the result of the singularity of a polar coordinate system (e.g.,

longitude is undefined at the north pole).  Encircling the north pole,

causes longitude to increase by 360<sup>o</sup>, as expected.  On the

other hand, encircling the south pole increases the longitude by

360<sup>o</sup> \e e = 36<sup>o</sup>.  The same behavior is exhibited

by the extended Gauss-Kr&uuml;ger projection.  The point latitude =

-1<sup>o</sup>, longitude = 90<sup>o</sup> projects to a point [2.65,1]

in Fig. 3(a).  If we now travel along this latitude line to longitude =

54<sup>o</sup> = 90<sup>o</sup> - 36<sup>o</sup> we arrive back at the

same projected position.  In the extended domain, the Thompson and the

Gauss-Kr&uuml;ger projections are not tightly tied to the globe but can

slip around it longitudinally.

\section testmerc Test data for the transverse Mercator projection

A test set for the transverse Mercator projection is available at

- <a href="http://sourceforge.net/projects/geographiclib/files/testdata/TMcoords.dat.gz/download">

TMcoords.dat.gz</a>

.

This is about 17 MB (compressed).  This test set consists of a set of

geographic coordinates together with the corresponding transverse

Mercator coordinates.  The WGS84 ellipsoid is used, with central

meridian 0<sup>o</sup>, central scale factor 0.9996 (the UTM value),

false easting = false northing = 0 m.

Each line of the test set gives 6 space delimited numbers

- latitude (degrees, exact)

- longitude (degrees, exact &mdash; see below)

- easting (meters, accurate to 0.1 pm)

- northing (meters, accurate to 0.1 pm)

- meridian convergence (degrees, accurate to 10<sup>-18</sup> deg)

- scale (accurate to 10<sup>-20</sup>)

.

The latitude and longitude are all multiples of 10<sup>-12</sup> deg and

should be regarded as exact, except that longitude =

82.63627282416406551 should be interpreted as exactly 90 (1 - \e e)

degrees.  These results are computed using Lee's formulas with

<a href="http://en.wikipedia.org/wiki/Maxima_(software)">Maxima</a>'s

bfloats and fpprec set to 80 (so the errors in the data are probably 1/2

of the values quoted above).  The Maxima code,

<a href="tm.mac">tm.mac</a> and <a href="ellint.mac">ellint.mac</a>,

used to prepare this data set is included in the distribution.  You will

need to have Maxima installed to use this code.  The comments at the top

of <a href="tm.mac">tm.mac</a> illustrate how to run it.

The contents of the file are as follows:

- 250000 entries randomly distributed in lat in [0, 90], lon in [0, 90];

- 1000 entries randomly distributed on lat in [0, 90], lon = 0;

- 1000 entries randomly distributed on lat = 0, lon in [0, 90];

- 1000 entries randomly distributed on lat in [0, 90], lon = 90;

- 1000 entries close to lat = 90 with lon in [0, 90];

- 1000 entries close to lat = 0, lon = 0 with lat >= 0, lon >= 0;

- 1000 entries close to lat = 0, lon = 90 with lat >= 0, lon <= 90;

- 2000 entries close to lat = 0, lon = 90 (1 - \e e) with lat >= 0;

- 25000 entries randomly distributed in lat

in [-89, 0], lon in [90 (1 - \e e), 90];

- 1000 entries randomly distributed on lat in [-89, 0], lon = 90;

- 1000 entries randomly distributed on lat in [-89, 0], lon = 90 (1 - \e e);

- 1000 entries close to lat = 0, lon = 90 (lat < 0, lon <= 90);

- 1000 entries close to lat = 0, lon = 90 (1 - \e e)

(lat < 0, lon <= 90 (1 - \e e));

.

(a total of 287000 entries).  The entries for lat < 0<sup>o</sup> and

lon in [90<sup>o</sup> (1 - \e e), 90<sup>o</sup>] use the "extended"

domain for the transverse Mercator projection explained in \ref extend.

The first 258000 entries have lat >= 0<sup>o</sup> and are suitable for

testing implementations following the standard convention.

The errors in the forward and reverse projections are converted into a

ground distance (e.g., by dividing the error in the projection by the

scale factor).  The errors in the convergence and in the scale may be

large because these quantities vary rapidly near singular points (for

convergence at the lat = 90<sup>o</sup> and for both convergence and

scale at lat = 0<sup>o</sup>, lon = 90<sup>o</sup> (1 - \e e)).  We

account for this by expressing the error as the sum of a constant and a

term proportional to the gradient of the quantity being measured.

\section tmerrors Accuracy of transverse Mercator projection

Errors in GeographicLib::TransverseMercatorExact.  For both Forward and

Reverse projections

- error in position < 9 nm (5 pm for long doubles)

.

These errors apply with the standard convention for the transverse

Mercator projection.  In this case, we use only test samples with

non-negative values of latitude.

For the extended transverse Mercator projection (Fig. 3(a)), the errors

given above apply only for latitude >= -15<sup>o</sup>.  For more negative

latitudes, the errors become large.  For example for latitude >=

-58<sup>o</sup>, the maximum error is 1 mm (and the scale has become 124

million).  This increase in the error is due to the use of the Thompson

projection in the computation.  Looking at the graticule for this

projection (Fig. 3(c)), we see that the Thompson scale becomes small close

to the south pole resulting in errors (measured as a ground distance)

which diverges (the errors vary inversely with scale).  For example at

lat = -58<sup>o</sup>, the scale for the Thompson projection is about

240000 times smaller than on the central meridian and the errors have

increased by about the same factor.  If necessary, this problem could be

avoided by reformulating the Thompson projection to place the origin at

the South pole.

If GeographicLib::TransverseMercatorExact is modified to use long

doubles (a 64-bit floating fraction) instead of of doubles (53-bit

floating fraction) then the error in the position is about 5 pm, for

latitude >= -15<sup>o</sup>.

Errors in GeographicLib::TransverseMercator.  For both Forward and

Reverse projections within 3900 km using the 6th-order series

(TM_TX_MAXPOW = 6, the default)

- error in position < 5 nm

.

Further from the central meridian, the errors grow rapidly.  For |\e x|

= 10700 km (approx 68<sup>o</sup> from the central median), the error in

position is about 1 mm.  When used in the UTM system, \e x is typically

restricted to |\e x| <= 500 km.  Within |\e x| <= 500 km, the errors

using the 6th-order series are dominated by round-off rather than the

number of terms.  If GeographicLib::TransverseMercator is modified to

use long double instead of double, the error in the position within the

the UTM region is about 2 pm.

If GeographicLib::TransverseMercator is compiled with TM_TX_MAXPOW = 4

to give the 4th-order approximation, then the error is about 210 nm

for |\e x| <= 500 km (the region used for UTM coordinates).

\section tmseries Series approximation for transverse Mercator

Kr&uuml;ger (1912) gives a 4th-order approximation to the transverse

Mercator projection.  The method entails performing first a

Gauss-Schreiber projection and approximating the warp of this to

Gauss-Kr&uuml;ger projection as a series in \e n = (\e a - \e b)/(\e a +

\e b).  This is accurate to about 200 nm within the UTM domain.  Here we

present the series extended to 10th order.  The series expanded to order

<i>n</i><sup>30</sup> are given in <a href="tmseries30.html">

tmseries30.html</a>.

By default, GeographicLib::TransverseMercator uses the 6th-order

approximation. The preprocessor variable TM_TX_MAXPOW can be used to

select an order from 4 thru 8.

In the formulas below ^ indicates exponentiation (\e n^3 = \e n*\e n*\e

n) and / indicates real division (3/5 = 0.6).  The equations need to be

converted to Horner form, but are here left in expanded form so that

they can be easily truncated to lower order in \e n.  Some of the

integers here are not representable as 32-bit integers and will need to

be included as double literals.

\e A in Kr&uuml;ger, p. 12, eq. (5)

\verbatim

A = a/(n + 1) * (1 + 1/4 * n^2

+ 1/64 * n^4

+ 1/256 * n^6

+ 25/16384 * n^8

+ 49/65536 * n^10);

\endverbatim

\e gamma in Kr&uuml;ger, p. 21, eq. (41)

\verbatim

alpha[1] =   1/2 * n

- 2/3 * n^2

+ 5/16 * n^3

+ 41/180 * n^4

- 127/288 * n^5

+ 7891/37800 * n^6

+ 72161/387072 * n^7

- 18975107/50803200 * n^8

+ 60193001/290304000 * n^9

+ 134592031/1026432000 * n^10;

alpha[2] =   13/48 * n^2

- 3/5 * n^3

+ 557/1440 * n^4

+ 281/630 * n^5

- 1983433/1935360 * n^6

+ 13769/28800 * n^7

+ 148003883/174182400 * n^8

- 705286231/465696000 * n^9

+ 1703267974087/3218890752000 * n^10;

alpha[3] =   61/240 * n^3

- 103/140 * n^4

+ 15061/26880 * n^5

+ 167603/181440 * n^6

- 67102379/29030400 * n^7

+ 79682431/79833600 * n^8

+ 6304945039/2128896000 * n^9

- 6601904925257/1307674368000 * n^10;

alpha[4] =   49561/161280 * n^4

- 179/168 * n^5

+ 6601661/7257600 * n^6

+ 97445/49896 * n^7

- 40176129013/7664025600 * n^8

+ 138471097/66528000 * n^9

+ 48087451385201/5230697472000 * n^10;

alpha[5] =   34729/80640 * n^5

- 3418889/1995840 * n^6

+ 14644087/9123840 * n^7

+ 2605413599/622702080 * n^8

- 31015475399/2583060480 * n^9

+ 5820486440369/1307674368000 * n^10;

alpha[6] =   212378941/319334400 * n^6

- 30705481/10378368 * n^7

+ 175214326799/58118860800 * n^8

+ 870492877/96096000 * n^9

- 1328004581729009/47823519744000 * n^10;

alpha[7] =   1522256789/1383782400 * n^7

- 16759934899/3113510400 * n^8

+ 1315149374443/221405184000 * n^9

+ 71809987837451/3629463552000 * n^10;

alpha[8] =   1424729850961/743921418240 * n^8

- 256783708069/25204608000 * n^9

+ 2468749292989891/203249958912000 * n^10;

alpha[9] =   21091646195357/6080126976000 * n^9

- 67196182138355857/3379030566912000 * n^10;

alpha[10]=   77911515623232821/12014330904576000 * n^10;

\endverbatim

\e beta in Kr&uuml;ger, p. 18, eq. (26*)

\verbatim

beta[1] =   1/2 * n

- 2/3 * n^2

+ 37/96 * n^3

- 1/360 * n^4

- 81/512 * n^5

+ 96199/604800 * n^6

- 5406467/38707200 * n^7

+ 7944359/67737600 * n^8

- 7378753979/97542144000 * n^9

+ 25123531261/804722688000 * n^10;

beta[2] =   1/48 * n^2

+ 1/15 * n^3

- 437/1440 * n^4

+ 46/105 * n^5

- 1118711/3870720 * n^6

+ 51841/1209600 * n^7

+ 24749483/348364800 * n^8

- 115295683/1397088000 * n^9

+ 5487737251099/51502252032000 * n^10;

beta[3] =   17/480 * n^3

- 37/840 * n^4

- 209/4480 * n^5

+ 5569/90720 * n^6

+ 9261899/58060800 * n^7

- 6457463/17740800 * n^8

+ 2473691167/9289728000 * n^9

- 852549456029/20922789888000 * n^10;

beta[4] =   4397/161280 * n^4

- 11/504 * n^5

- 830251/7257600 * n^6

+ 466511/2494800 * n^7

+ 324154477/7664025600 * n^8

- 937932223/3891888000 * n^9

- 89112264211/5230697472000 * n^10;

beta[5] =   4583/161280 * n^5

- 108847/3991680 * n^6

- 8005831/63866880 * n^7

+ 22894433/124540416 * n^8

+ 112731569449/557941063680 * n^9

- 5391039814733/10461394944000 * n^10;

beta[6] =   20648693/638668800 * n^6

- 16363163/518918400 * n^7

- 2204645983/12915302400 * n^8

+ 4543317553/18162144000 * n^9

+ 54894890298749/167382319104000 * n^10;

beta[7] =   219941297/5535129600 * n^7

- 497323811/12454041600 * n^8

- 79431132943/332107776000 * n^9

+ 4346429528407/12703122432000 * n^10;

beta[8] =   191773887257/3719607091200 * n^8

- 17822319343/336825216000 * n^9

- 497155444501631/1422749712384000 * n^10;

beta[9] =   11025641854267/158083301376000 * n^9

- 492293158444691/6758061133824000 * n^10;

beta[10]=   7028504530429621/72085985427456000 * n^10;

\endverbatim

The following table gives the truncation error when the series above are

truncated to order \e n<sup><i>J</i></sup>.  The error is the maximum of

the errors for the forward and reverse projections, converted in both

cases to a true distance (in meters).  It is expressed as a function of

the geodesic distance (in km) from the central meridian.  This table was

computed for the WGS84 ellipsoid.  When implementing with floating point

arithmetic on a computer, there is an additional rounding error incurred;

this adds 2.2 m for floats, 4.9 nm for doubles and 2.0 pm for long

doubles.

\verbatim

dist J=2     J=4     J=5     J=6     J=7     J=8     J=10    J=12

0 3.0e-02 1.9e-07 5.9e-10 1.7e-12 6.6e-15 2.4e-17 3.5e-22 5.6e-27

500 3.2e-02 2.1e-07 7.3e-10 2.4e-12 9.4e-15 3.7e-17 6.6e-22 1.3e-26

1000 3.7e-02 3.1e-07 1.2e-09 4.8e-12 2.1e-14 9.8e-17 2.3e-21 5.9e-26

1500 4.7e-02 5.3e-07 2.4e-09 1.1e-11 5.8e-14 3.1e-16 9.4e-21 3.2e-25

2000 6.3e-02 1.0e-06 5.3e-09 2.9e-11 1.7e-13 1.0e-15 4.3e-20 1.9e-24

2500 8.8e-02 2.1e-06 1.2e-08 8.0e-11 5.4e-13 3.8e-15 2.1e-19 1.3e-23

3000 1.3e-01 4.5e-06 3.1e-08 2.3e-10 1.9e-12 1.5e-14 1.2e-18 9.9e-23

3500 1.9e-01 9.6e-06 8.0e-08 7.2e-10 6.8e-12 6.8e-14 7.3e-18 8.6e-22

4000 3.0e-01 2.1e-05 2.2e-07 2.4e-09 2.7e-11 3.2e-13 5.0e-17 8.6e-21

4500 4.7e-01 5.0e-05 6.2e-07 8.3e-09 1.2e-10 1.7e-12 3.9e-16 1.0e-19

5000 7.7e-01 1.3e-04 1.9e-06 3.2e-08 5.5e-10 9.9e-12 3.5e-15 1.4e-18

5500 1.3e+00 3.4e-04 6.6e-06 1.4e-07 3.0e-09 6.8e-11 3.8e-14 2.3e-17

6000 2.4e+00 1.0e-03 2.6e-05 6.8e-07 1.9e-08 5.6e-10 5.2e-13 5.3e-16

6500 4.7e+00 3.6e-03 1.2e-04 4.1e-06 1.5e-07 5.9e-09 9.6e-12 1.7e-14

7000 1.0e+01 1.5e-02 6.6e-04 3.2e-05 1.6e-06 8.7e-08 2.7e-10 9.1e-13

7500 2.5e+01 7.7e-02 5.0e-03 3.5e-04 2.6e-05 2.0e-06 1.3e-08 9.4e-11

8000 7.6e+01 5.7e-01 5.9e-02 6.5e-03 7.6e-04 9.2e-05 1.5e-06 2.6e-08

8500 3.3e+02 8.0e+00 1.5e+00 2.9e-01 5.9e-02 1.3e-02 6.3e-04 3.4e-05

9000 2.9e+03 3.9e+02 1.6e+02 7.4e+01 3.5e+01 1.7e+01 4.4e+00 1.2e+00

\endverbatim

<center>

<img src="gauss-krueger-error.png"

width=672 height=552 alt="Truncation error for Gauss-Krueger series">

<a href="gauss-krueger-error.pdf">Fig. 2</a>

</center>\n

The truncation error for the series for the Gauss-Kr&uuml;ger transverse

Mercator projection.  The blue curves show the truncation error for the

order of the series \e J = 2 (top) thru \e J = 12 (bottom).  The red

curves show the combined truncation and round-off errors for

- float and \e J = 4 (top)

- double and \e J = 6 (middle)

- long double and \e J = 8 (bottom)

The high-order expansions for \e alpha and \e beta were produced by the

Maxima program <a href="tmseries.mac">tmseries.mac</a> (included in the

distribution).  To run, start Maxima and enter

\verbatim

load("tmseries.mac")$

\endverbatim

Further instructions are included at the top of the file.

<center>

Back to \ref organization.  Forward to \ref geodesic.  Up to \ref contents.

</center>

**********************************************************************/

/**

\page geodesic Geodesics on the Ellipsoid

<center>

Back to \ref transversemercator.  Forward to \ref geocentric.  Up to \ref contents.

</center>

GeographicLib::Geodesic and GeographicLib::GeodesicLine provide accurate

solutions to the direct and inverse geodesic problems.  The \ref geod

utility provides an interface to these classes.

GeographicLib::AzimuthalEquidistant implements the azimuthal equidistant

projection in terms of geodesics.  GeographicLib::CassiniSoldner

implements a transverse cylindrical equidistant projection in terms of

geodesics.  The \ref equidistanttest utility provides an interface to

these projections.

References

- F. W. Bessel,

<a href="http://adsabs.harvard.edu/abs/1825AN......4..241B">&Uuml;ber die

Berechnung der geographischen L&auml;ngen und Breiten aus geod&auml;tischen

Vermessungen</a>,

Astron. Nachr. 4 (86), 241&ndash;254 (1826).  English translation:

<a href="http://arxiv.org/abs/0908.1824">arXiv:0908.1824</a>.

- F. R. Helmert,

Die Mathematischen und Physikalischen Theorieen der H&ouml;heren

Geod&auml;sie, vol. 1 (Teubner, Leipzig, 1880).

- E. B. Christoffel,

Allgemeine Theorie der geod&auml;tischen Dreiecke (1868),

in "Gesammelte Mathematische Abhandlungen",

vol. 1, chap. 16, pp. 297&ndash;346 (Teubner, Leipzig, 1910).

- H. F. Rainsford,

<a href="http://dx.doi.org/10.1007/BF02527187">

Long geodesics on the ellipsoid</a>,

Bull. Geod. 37 (1), 12&ndash;22 (1955).

- T. Vincenty,

<a href="http://www.ngs.noaa.gov/PUBS_LIB/inverse.pdf">Direct and Inverse

Solutions of Geodesics on the Ellipsoid with Application of Nested

Equations</a>,

Survey Review 23 (176), 88&ndash;93 (1975).

- A collection of some older papers on geodesics is available at

http://trac.osgeo.org/proj/wiki/GeodesicCalculations#TheHistory

\section geoddesc Description of algorithm

The shortest path between two points on a ellipsoid at (\e lat1, \e lon1)

and (\e lat2, \e lon2) is called the geodesic.  Its length is \e s12 and

the geodesic from point 1 to point 2 has azimuths \e azi1 and \e azi2 at

the two end points.  (The azimuth is the heading measured clockwise from

north.  \e azi2 is the "forward" azimuth, i.e., the heading that takes

you beyond point 2 not back to point 1.)  The reduced length of the

geodesic, \e m12, is the rate at which the second point moves as the

geodesic is rotated about the first point.  Christoffel (1868) proved

that this is equal to the rate at which the first point moves when the

geodesic is rotated about the second point.

Given \e lat1, \e lon1, \e azi1, and \e s12, we can determine \e lat2,

\e lon2, \e azi2, \e m12.  This is the <b>direct</b> geodesic problem.

(If \e s12 is sufficiently large that the geodesic wraps more than

halfway around the earth, there will be another geodesic between the

points with a smaller \e s12.)

Given \e lat1, \e lon1, \e lat2, and \e lon2, we can determine \e azi1,

\e azi2, \e s12, \e m12.  This is the <b>inverse</b> geodesic problem.

Usually, the solution to the inverse problem is unique.  In cases where

there are multiple solutions (all with the same \e s12, of course), all

the solutions can be easily generated once a particular solution is

provided.

The framework for solving geodesic problems on the ellipsoid was laid

down by Bessel (1826) who showed how the problem may be transferred to

an "auxiliary sphere" where the latitude \e phi has been replaced by the

reduced latitude \e beta where tan(\e beta) = (1 - \e f) tan(\e phi).

On this sphere, the geodesic is a great circle and the azimuth is the

same as on the ellipsoid.  However, the ellipsoidal distance is related to

the great circle distance by an integral; and the ellipsoidal longitude

is similarly related to the longitude on the auxiliary sphere.  In the

case of small \e f, Bessel developed series expansions for the integrals

and provided a systematic solution to the direct problem.  Helmert

(1880) extended Bessel's treatment by giving an expression for the

reduced length of a geodesic for the ellipsoid.  The "azimuthal scale" of

the azimuthal equidistant projection about point 1 is given by \e s12/\e

m12.

The algorithms included here are based on the work of Bessel (1826) and

Helmert (1880).  Key points are:

- Extension of Bessel's method for removing half the terms in the

series expansions to the longitude integral.  This also accelerates

the convergence of the series.

- Inclusion of sufficient terms in the series to give accuracy at the

round-off level for doubles and long doubles.

- Reverting the distance series so that the direct problem can be

solved non-iteratively.

- Calculation of the reduced length for the geodesic.

- Solution the inverse problem via Newton's method, using the reduced

length to compute the needed derivative.

.

Vincenty (1975) has provided algorithms for the direct and inverse

geodesic problems based in the series given by Rainsford (1955) (based,

in turn, on Bessel).  However Vincenty's methods were designed for use

on the desk calculators of that era and are not optimal (in terms of

accuracy and robustness) for a general purpose computer.  The

distinguishing features of the algorithms presented here compared to

Vincenty's are:

- Accuracy to close to the round-off limit (i.e., 15 nm for doubles).

- Complete solution of the inverse problem.  (Vincenty's inverse

algorithm sometimes fails.)

- Return the reduced length of the geodesic.

The series expansions for the integrals are obtained using <a

href="http://en.wikipedia.org/wiki/Maxima_(software)">Maxima</a>.  In

this release, we use a 6th-order expansions.  This is sufficient to

maintain accuracy for doubles for the SRMmax ellipsoid (\e a = 6400 km,

\e f = 1/150).  However, the preprocessor macro GEOD_ORD can be used to

select any order up to 8.  (If using long doubles, with a 64-bit

fraction, the default order is 7.)  The series is evaluated by a

combination of Horner's method and Clenshaw summation.  For the direct

problem, we use a reversion of the series for the distance to compute

the spherical arc length and thereby complete the solution without the

need to iterate.

The Maxima code, <a href="geod.mac">geod.mac</a>, used to derive the

series for the distance and longitude integrals.  In addition this

includes code to format the results for inclusion into C++ code.  You

will need to have Maxima installed to use this code.  The comments at

the top of <a href="geod.mac">geod.mac</a> illustrate how to run it.  It

takes about 10s to generate the 8th order series.

For the inverse problem, we rearrange the points so that the point 1 is

the one closest to a pole and is in the southern hemisphere and the

point 2 is to the north and east of point 1.  We consider geodesics

leaving point 1 with azimuth \e alpha1 in [0, pi] and we determine the

longitude where these geodesics first intersect the latitude = \e lat2.

The longitude is an increasing function of \e alpha1 and we use Newton's

method to determine the value of \e alpha1 for which the longitude

matches \e lon2.  This then gives the desired geodesic.

This class also returns the arc length \e a12 on the auxiliary sphere

and allows distances to be specified in these terms.  See the -a option

to \ref geod.

\section testgeod Test data for geodesics

A test set a geodesics is available at

- <a href="http://sourceforge.net/projects/geographiclib/files/testdata/GeodTest.dat.gz/download">

GeodTest.dat.gz</a>

.

This is about 34 MB (compressed).  This consists of a set of geodesics

for the WGS84 ellipsoid.  A subset of this (consisting of 1/50 of the

members &mdash; about 690 kB, compressed) is available at

- <a href="http://sourceforge.net/projects/geographiclib/files/testdata/GeodTest-short.dat.gz/download">

GeodTest-short.dat.gz</a>

Each line of the test set gives 9 space delimited numbers

- latitude for point 1, \e lat1 (degrees, exact)

- longitude for point 1, \e lon1 (degrees, always 0)

- azimuth for point 1, \e azi1 (clockwise from north in degrees, exact)

- latitude for point 2, \e lat2 (degrees, accurate to 10<sup>-18</sup> deg)

- longitude for point 2, \e lon2 (degrees, accurate to 10<sup>-18</sup> deg)

- azimuth for point 2, \e azi2 (degrees, accurate to 10<sup>-18</sup> deg)

- geodesic distance from point 1 to point 2, \e s12 (meters, exact)

- arc distance on the auxiliary sphere, \e a12 (degrees, accurate to

10<sup>-18</sup> deg)

- reduced length of the geodesic, \e m12 (meters, accurate to 0.1 pm)

.

These are computed using as direct geodesic calculations with the given

\e lat1, \e lon1, \e azi1, and \e s12.  The distance \e s12 always

corresponds to an arc length \e a12 <= 180<sup>o</sup>, so the given

geodesics give the shortest paths from point 1 to point 2.  For

simplicity and without loss of generality, \e lat1 is chosen in

[0<sup>o</sup>, 90<sup>o</sup>], \e lon1 is taken to be zero, \e azi1 is

chosen in [0<sup>o</sup>, 180<sup>o</sup>].  Furthermore, \e lat1 and \e

azi1 are taken to be multiples of 10<sup>-12</sup> deg and \e s12 is a

multiple of 0.1 um in [0 m, 20003931.4586254 m].  This results \e lon2

in [0<sup>o</sup>, 180<sup>o</sup>] and \e azi2 in [0<sup>o</sup>,

180<sup>o</sup>].

The direct calculation uses an expansion of the geodesic equations

accurate to \e f<sup>20</sup> (approximately 1 part in 10<sup>50</sup>)

and is computed with with

<a href="http://en.wikipedia.org/wiki/Maxima_(software)">Maxima</a>'s

bfloats and fpprec set to 100 (so the errors in the data are probably

1/2 of the values quoted above).

The contents of the file are as follows:

- 100000 entries randomly distributed

- 50000 entries which are nearly antipodal

- 50000 entries with short distances

- 50000 entries with one end near a pole

- 50000 entries with both ends near opposite poles

- 50000 entries which are nearly meridional

- 50000 entries which are nearly equatorial

- 50000 entries running between vertices (\e azi1 = \e azi2 = 90<sup>o</sup>)

- 50000 entries ending close to vertices

.

(a total of 500000 entries).  The values for \e s12 for the geodesics

running between vertices are truncated to a multiple of 0.1 pm and this

is used to determine point 2.

This data can be fed to the \ref geod utility as follows

- Direct from point 1:

\verbatim

gunzip -c GeodTest.dat.gz | cut -d' ' -f1,2,3,7 | ./Geod

\endverbatim

This should yield columns 4, 5, 6, and 9 of the test set.

- Direct from point 2:

\verbatim

gunzip -c GeodTest.dat.gz | cut -d' ' -f4,5,6,7 |

sed "s/ \([^ ]*$\)/ -\1/" | ./Geod

\endverbatim

(The sed command negates the distance.)  This should yield columns 1,

2, and 3, and the negative of column 9 of the test set.

- Inverse between points 1 and 2:

\verbatim

gunzip -c GeodTest.dat.gz | cut -d' ' -f1,2,4,5 | ./Geod -i

\endverbatim

This should yield columns 3, 6, 7, and 9 of the test set.

.

Add, e.g., "-p 6", to the call to Geod to change the precision of the

output.  Adding "-f" causes Geod to print all 9 fields specifying the

geodesic (in the same order as the test set).

\section geoderrors Accuracy of geodesic calculation

We regard all the test data as "exact".  For each member of the test

set, 6 error distances are computed.  (The direct problem is solved from

both ends and the maximum of the errors is used.)

-# The error in the position of point 2 for the direct problem.

-# Error in the direction of geodesic at point 2 for the direct

problem scaled by the major radius.  (This is the direction in 3D

space to allow canceling variations in longitude and azimuth near

the poles.)

-# Error in m12 for the direct problem.

-# Error in s12 for the inverse problem.

-# Error in the azimuths at points 1 and 2 for the inverse problem

scaled by m12.

-# Consistency of the azimuths at points 1 and 2 for the inverse

problem.  Compute geodesic from each endpoint using the computed

azimuths; compare positions on each geodesic at midpoint (for long

geodesics) or a distance equal to the major radius beyond each end

of the geodesic (for short geodesics).

.

The last check verifies that \e azi1 and \e azi2 returned by the inverse

solution correspond to the same path and is important, e.g., when point

1 and point 2 are near opposite poles.  To guard against a canceling

error, the calculations in last error estimate use a long double

version of the direct geodesic calculation (which is about 2000 times

more accurate than the double version).  The maximum of the resulting

errors in less than 15 nm for the double version and 7 pm for the long

double version.

\section geodseries Expansions for geodesics

We give here the series expansions for the various geodesic integrals

valid to order <i>f</i><sup>8</sup>.

In the formulas below ^ indicates exponentiation (\e f^3 = \e f*\e f*\e

f) and / indicates real division (3/5 = 0.6).  The equations need to be

converted to Horner form, but are here left in expanded form so that

they can be easily truncated to lower order.  These expansions were

obtained using the the Maxima code, <a href="geod.mac">geod.mac</a>.

The series expanded to order

<i>f</i><sup>30</sup> are given in <a href="geodseries30.html">

geodseries30.html</a>.

In the expansions below, we have

- \e alpha<sub>0</sub> is the azimuth at the equator crossing

- \e lambda is the longitude measured from the equator crossing

- \e omega is the spherical longitude

- \e sigma is the spherical arc length

- \e f is the flattening

- \e e<sup>2</sup> = \e f (2 - \e f)

- \e e'<sup>2</sup> = \e e<sup>2</sup>/(1 - \e e<sup>2</sup>)

- \e k<sup>2</sup> = \e e'<sup>2</sup> cos<sup>2</sup> \e alpha<sub>0</sub>

= 4 \e eps / (1 - \e eps)<sup>2</sup>

- \e del = (\e f - \e eps) / (1 - \e eps)

- \e nu = \e eps / \e del

The formula for distance is

&nbsp; &nbsp; \e s/\e b = \e I1(\e sigma)

where

&nbsp; &nbsp; \e I1(\e sigma) = \e A1 (\e sigma + \e B1(\e sigma))<br>

&nbsp; &nbsp; \e B1(\e sigma) =

sum<sub>\e j = 1</sub> \e C1<sub>j</sub> sin(2 \e j \e sigma)

and

\verbatim

A1 = (1 + 1/4 * eps^2

+ 1/64 * eps^4

+ 1/256 * eps^6

+ 25/16384 * eps^8) / (1 - eps);

\endverbatim

\verbatim

C1[1] = - 1/2 * eps

+ 3/16 * eps^3

- 1/32 * eps^5

+ 19/2048 * eps^7;

C1[2] = - 1/16 * eps^2

+ 1/32 * eps^4

- 9/2048 * eps^6

+ 7/4096 * eps^8;

C1[3] = - 1/48 * eps^3

+ 3/256 * eps^5

- 3/2048 * eps^7;

C1[4] = - 5/512 * eps^4

+ 3/512 * eps^6

- 11/16384 * eps^8;

C1[5] = - 7/1280 * eps^5

+ 7/2048 * eps^7;

C1[6] = - 7/2048 * eps^6

+ 9/4096 * eps^8;

C1[7] = - 33/14336 * eps^7;

C1[8] = - 429/262144 * eps^8;

\endverbatim

The function \e tau(\e sigma) = \e s/(\e b \e A1) = \e sigma + \e B1(\e

sigma) may be inverted by series reversion giving

&nbsp; &nbsp; \e sigma(\e tau) =

\e tau + sum<sub>\e j = 1</sub> \e C1'<sub>j</sub> sin(2 \e j \e tau)

where

\verbatim

C1'[1] = + 1/2 * eps

- 9/32 * eps^3

+ 205/1536 * eps^5

- 4879/73728 * eps^7;

C1'[2] = + 5/16 * eps^2

- 37/96 * eps^4

+ 1335/4096 * eps^6

- 86171/368640 * eps^8;

C1'[3] = + 29/96 * eps^3

- 75/128 * eps^5

+ 2901/4096 * eps^7;

C1'[4] = + 539/1536 * eps^4

- 2391/2560 * eps^6

+ 1082857/737280 * eps^8;

C1'[5] = + 3467/7680 * eps^5

- 28223/18432 * eps^7;

C1'[6] = + 38081/61440 * eps^6

- 733437/286720 * eps^8;

C1'[7] = + 459485/516096 * eps^7;

C1'[8] = + 109167851/82575360 * eps^8;

\endverbatim

The reduced length is given by

&nbsp; &nbsp; \e m/\e b =

sqrt(1 + \e k<sup>2</sup> sin<sup>2</sup>\e sigma<sub>2</sub>)

cos \e sigma<sub>1</sub> sin \e sigma<sub>2</sub><br>

&nbsp; &nbsp; &nbsp; &nbsp; &nbsp; &nbsp; -

sqrt(1 + \e k<sup>2</sup> sin<sup>2</sup>\e sigma<sub>1</sub>)

sin \e sigma<sub>1</sub> cos \e sigma<sub>2</sub><br>

&nbsp; &nbsp; &nbsp; &nbsp; &nbsp; &nbsp; -

cos \e sigma<sub>1</sub>  cos \e sigma<sub>2</sub>

(\e J(\e sigma<sub>2</sub>) - \e J(\e sigma<sub>1</sub>))

where

&nbsp; &nbsp; \e J(\e sigma) =

\e I1(\e sigma) - \e I2(\e sigma)<br>

&nbsp; &nbsp; \e I2(\e sigma) = \e A2 (\e sigma + \e B2(\e sigma))<br>

&nbsp; &nbsp; \e B2(\e sigma) =

sum<sub>\e j = 1</sub> \e C2<sub>j</sub> sin(2 \e j \e sigma)

\verbatim

A2 = (1 + 1/4 * eps^2

+ 9/64 * eps^4

+ 25/256 * eps^6

+ 1225/16384 * eps^8) * (1 - eps);

\endverbatim

\verbatim

C2[1] = + 1/2 * eps

+ 1/16 * eps^3

+ 1/32 * eps^5

+ 41/2048 * eps^7;

C2[2] = + 3/16 * eps^2

+ 1/32 * eps^4

+ 35/2048 * eps^6

+ 47/4096 * eps^8;

C2[3] = + 5/48 * eps^3

+ 5/256 * eps^5

+ 23/2048 * eps^7;

C2[4] = + 35/512 * eps^4

+ 7/512 * eps^6

+ 133/16384 * eps^8;

C2[5] = + 63/1280 * eps^5

+ 21/2048 * eps^7;

C2[6] = + 77/2048 * eps^6

+ 33/4096 * eps^8;

C2[7] = + 429/14336 * eps^7;

C2[8] = + 6435/262144 * eps^8;

\endverbatim

The longitude is given in terms of the spherical longitude by

&nbsp; &nbsp;

\e lambda = \e omega - \e f sin \e alpha<sub>0</sub> \e I3(\e sigma)

where

&nbsp; &nbsp; \e I3(\e sigma) = \e A3 (\e sigma + \e B3(\e sigma))<br>

&nbsp; &nbsp; \e B3(\e sigma) =

sum<sub>\e j = 1</sub> \e C3<sub>j</sub> sin(2 \e j \e sigma)

and

\verbatim

A3 = (2 - f) / (2 - del) * (1

- 1/16 * nu^2 * del^3

- 1/64 * nu^4 * del^4

+ 1/64 * nu^2 * del^5

+ (1/64 * nu^2 + 1/64 * nu^4 - 1/128 * nu^6) * del^6

+ (3/256 * nu^2 + 7/1024 * nu^4 + 15/2048 * nu^6) * del^7);

\endverbatim

\verbatim

C3[1] = + 1/4 * eps

- 1/8 * eps*del

- (1/16 + 1/64 * nu^2) * eps*del^2

- (1/32 + 1/128 * nu^2) * eps*del^3

- (1/64 - 1/128 * nu^2 + 1/128 * nu^4) * eps*del^4

- (1/128 - 1/128 * nu^2 - 3/1024 * nu^4) * eps*del^5

- (1/256 - 5/1024 * nu^2 - 5/1024 * nu^4 + 61/16384 * nu^6) * eps*del^6;

C3[2] = + 1/16 * eps^2

- 3/64 * eps^2*del

- (1/64 + 1/128 * nu^2) * eps^2*del^2

- 1/256 * eps^2*del^3

+ (5/1024 * nu^2 - 7/2048 * nu^4) * eps^2*del^4

+ (1/1024 + 3/1024 * nu^2 + 37/16384 * nu^4) * eps^2*del^5;

C3[3] = + 5/192 * eps^3

- 3/128 * eps^3*del

- (1/192 + 7/1536 * nu^2) * eps^3*del^2

+ 1/768 * nu^2 * eps^3*del^3

+ (1/1024 + 35/12288 * nu^2 - 91/49152 * nu^4) * eps^3*del^4;

C3[4] = + 7/512 * eps^4

- 7/512 * eps^4*del

- (1/512 + 3/1024 * nu^2) * eps^4*del^2

+ (5/8192 + 23/16384 * nu^2) * eps^4*del^3;

C3[5] = + 21/2560 * eps^5

- 9/1024 * eps^5*del

- (3/4096 + 33/16384 * nu^2) * eps^5*del^2;

C3[6] = + 11/2048 * eps^6

- 99/16384 * eps^6*del;

C3[7] = + 429/114688 * eps^7;

\endverbatim

\section geodmisc Miscellaneous information on geodesics

When either point is at a pole, the azimuth is defined by keeping the

longitude fixed and writing \e lat = 90 - \e eps or -90 + \e eps and

taking the limit \e eps -> 0 from above.

The following prescription allows you to ascertain when there are

multiple solutions to the inverse problem and to generate the complete

set.

- \e lat1 = \e -lat2 (with neither at a pole).  If \e azi1 = \e azi2,

the geodesic is unique.  Otherwise there are two geodesics and the

second one is obtained by setting [\e azi1, \e azi2] = [\e azi2, \e

azi1].  (This occurs when the longitude difference is near +/-

180<sup>o</sup> for oblate ellipsoids.)

- \e lon2 = \e lon1 +/- 180<sup>o</sup> (with neither at a pole).  If \e

azi1 = 0<sup>o</sup> or +/- 180<sup>o</sup>, the geodesic is unique.

Otherwise there are two geodesics and the second one is obtained by

setting [\e azi1, \e azi2] = [-\e azi1, -\e azi2].  (This occurs when

the \e lat2 is near -\e lat1 for prolate ellipsoids.)

- Points 1 and 2 at opposite poles.  There are infinitely many

geodesics which can be generated by setting [\e azi1, \e azi2] = [\e

azi1, \e azi2] + [\e d, -\e d], for arbitrary \e d.  (For spheres,

this prescription applies when points 1 and 2 are antipodal.)

- \e s12 = 0 (coincident points).  There are infinitely many geodesics

which can be generated by setting [\e azi1, \e azi2] = [\e azi1, \e

azi2] + [\e d, \e d], for arbitrary \e d.

A rather large fraction of the code in GeographicLib::Geodesic deals

merely with the spherical trigonometry required to do great circle

calculations.  When high accuracy is required, care has to be taken to

use stable methods for computing angles.  Typically, we represent angles

internally with the pair [sin(\e theta), cos(\e theta)].  This expands

the number of representable angles by about 4; in particular, it allows

angles very close to all the cardinal points to be resolved.  We use

expressions for sin(\e theta) and cos(\e theta) which avoid large

cancellations.  Having a comprehensive test set as described in \ref

testgeod helps to identify problems.  This representation allows input

arguments which are multiples of 90<sup>o</sup> to be represented

exactly.

The code for the inverse treats equatorial and meridional geodesics

specially.  All other case are handled by the same code (implementing

Newton's method) with tests to select appropriate starting guess in

difference regimes.  Specifying a zero inverse flattening in Geodesic

constructor is equivalent to zero flattening (i.e., a sphere).  The

method accommodates this case without any special coding.

In order to limits the enumeration of cases, the inverse problem is

brought to a canonical form by interchanging the points and changing the

signs of the coordinates.  The effect of these changes are undone at the

end by applying sign changes to the final calls to atan2 to give the

azimuths.

The derivative needed for Newton's method for the inverse problem is

given in terms of the reduced length.  The reduced length can also be

used to apply Newton's method to solve problems such as finding the

distance between a point and a geodesic line.

Convergence failures with Newton's method for the inverse problem is

signaled by negating the distance (and the arc distance and reduced

length) and by reversing the azimuths.  This convention was chosen as it

allows both detection of the problem and subsequent use of the values

(assuming that they are approximately correct).  It is expected that the

method converges for all models of the earth and for any two input

points. Please report all cases where this does not happen.

Geographic coordinates which are close to zero are quantized (by adding

and subtracting 1/16 deg).  This allows certain cases of underflow to be

avoided.  This limits the accuracy to which coordinates can be specified

to 0.7 pm.  This should not be a problem since the overall accuracy of

the algorithms is only 15 nm.

You can specify a \e negative flattening to get a prolate ellipsoid.  The

rule is that \e b is the radius of the axis of symmetry (the polar

radius), that \e a is the equatorial radius, and that \e b = \e a (1 -

\e f).  The direct solution needed no modification.  The inverse

solution required a generalization in the way starting points for

Newton's method are chosen; in addition a test are added to the case

when the endpoints are on the same meridian to determine if a shorter

non-meridional geodesic exists.

Most planets and satellites are approximated by oblate ellipsoids.

However, satellites whose rotation frequency is tidally locked to a

month may be prolate (as example is Saturn's moon, Tethys).  Such

satellites are typically considerably more eccentric that the earth and

it may be necessary to increase the order of the series used in order to

maintain accuracy.

GeodesicLine::Position provides a way of doing a series of direct

calculations for a single geodesic.  This is about 2.1 times faster than

calling Geodesic::Direct.  A speed-up by a further factor of 1.6 is

obtained with the direct calculation if the distance is expressed in

terms of the spherical arc length.

On a 2.6 GHz Intel machine (g++, version 4.3.0, -O3), timings are as

follows:

- Geodesic::Direct 0.93 us

- GeodesicLine::Position 0.43 us (arcmode = false)

- GeodesicLine::Position 0.27 us (arcmode = true)

- Geodesic::Inverse 2.6 us

.

The timing for Geodesic::Inverse is a mean.  Individual calls may take a

few times longer because of the need to do additional Newton iterations.

Mean number of Newton iterations is about 3, but this increases (rarely)

to 17 with some values of the arguments.

Future work:

- More documentation.

<center>

Back to \ref transversemercator.  Forward to \ref geocentric.  Up to \ref contents.

</center>

**********************************************************************/

/**

\page geocentric Geocentric coordinates

<center>

Back to \ref geodesic.  Forward to \ref geoid.  Up to \ref contents.

</center>

The implementation of GeographicLib::Geocentric::Reverse is adapted from

- H. Vermeille,

<a href="http://dx.doi.org/10.1007/s00190-002-0273-6">

Direct transformation from geocentric coordinates to geodetic

coordinates</a>, J. Geodesy 76, 451&ndash;454 (2002).

This provides a closed-form solution but can't directly be applied close to

the center of the earth.  Several changes have been made to remove this

restriction and to improve the numerical accuracy.  Now the method is

accurate for all inputs (even if \e h is infinite).

The problems encountered near the center of the ellipsoid are:

- There's a potential division by zero in the definition of \e s. The

equations are easily reformulated to avoid this problem.

- \e t<sup>3</sup> may be negative.  This is OK; we just take the real root.

- The solution for \e t may be complex.  However this leads to 3 real roots

for \e u/\e r.  It's then just a matter of picking the one that computes

the geodetic result which minimizes |\e h| and which avoids large

round-off errors.

- Some of the equations result in a large loss of accuracy due to

subtracting nearly equal quantities.  E.g., \e k= sqrt(\e u + \e v + \e

w<sup>2</sup>) - \e w is inaccurate if \e u + \e v is small; we can fix

this by writing \e k = (\e u + \e v)/(sqrt(\e u + \e v + \e w<sup>2</sup>)

+ \e w).

The error is computed as follows.  Write a version of

Geocentric::WGS84.Forward which uses long doubles (including using long

doubles for the WGS84 parameters).  Generate random (long double) geodetic

coordinates (\e lat0, \e lon0, \e h0) and use the "long double"

WGS84.Forward to obtain the corresponding (long double) geocentric

coordinates (\e x0, \e y0, \e z0).  [We restrict \e h0 so that \e h0 >= - \e

a (1 - \e e<sup>2</sup>) / sqrt(1 - \e e<sup>2</sup> sin<sup>2</sup>\e

lat0), which ensures that (\e lat0, \e lon0, \e h0) is the principal

geodetic inverse of (\e x0, \e y0, \e z0).]  Because the forward calculation

is numerically stable and because long doubles (on Linux systems using g++)

provide 11 bits additional accuracy (about 3.3 decimal digits), we regard

this set of test data as exact.

Apply the double version of WGS84.Reverse to (\e x0, \e y0, \e z0) to

compute the approximate geodetic coordinates (\e lat1, \e lon1, \e h1).

Convert (\e lat1 - \e lat0, \e lon1 - \e lon0) to a distance, \e ds, on the

surface of the ellipsoid and define \e err = hypot(\e ds, \e h1 - \e h0).

For |\e h0| < 5000 km, we have \e err < 7 nm.

This methodology is not very useful very far from the globe, because the

absolute errors in the approximate geodetic height become large, or within

50 km of the center of the earth, because of errors in computing the

approximate geodetic latitude.  To illustrate the second issue, the maximum

value of \e err for \e h0 < 0 is about 80 mm.  The error is maximum close to

the circle given by geocentric coordinates satisfying hypot(\e x, \e y) = \e

a \e e<sup>2</sup> (= 42.7 km), \e z = 0.  (This is the center of meridional

curvature for \e lat = 0.)  The geodetic latitude for these points is \e lat

= 0.  However, if we move 1 nm towards the center of the earth, the geodetic

latitude becomes 0.04", a distance of 1.4 m from the equator.  If, instead,

we move 1 nm up, the geodetic latitude becomes 7.45", a distance of 229 m

from the equator.  In light of this, Reverse does quite well in this

vicinity.

To obtain a practical measure of the error for the general case we define

- <i>err</i><sub>h</sub> = |\e h1 - \e h0| / max(1, \e h0 / \e a)

- for \e h0 > 0, <i>err</i><sub>out</sub> = \e ds

- for \e h0 < 0, apply the long double version of WGS84.Forward to (\e lat1,

\e lon1, \e h1) to give (\e x1, \e y1, \e z1) and compute

<i>err</i><sub>in</sub> = hypot(\e x1 - \e x0, \e y1 - \e y0, \e z1 - \e

z0).

.

We then find <i>err</i><sub>h</sub> < 8 nm, <i>err</i><sub>out</sub> < 4 nm,

and <i>err</i><sub>in</sub> < 7 nm.

The testing has been confined to the WGS84 ellipsoid.  The method will work

for all ellipsoids used in terrestrial geodesy.  However, the central region,

which leads to multiple real roots for the cubic equation in Reverse, pokes

outside the ellipsoid (at the poles) for ellipsoids with \e e > 1/sqrt(2).

Reverse has not been analysed for this case.  Similarly ellipsoids which are

very nearly spherical near yield inaccurate results due to underflow; in the

other hand, the case of the sphere, \e f = 0, is treated specially and gives

accurate results.

Other comparable methods are K. M. Borkowski,

<a href="http://dx.doi.org/10.1007/BF00643807"> Transformation

of geocentric to geodetic coordinates without approximations</a>,

Astrophys. Space Sci. 139, 1&ndash;4 (1987)

(<a href="http://dx.doi.org/10.1007/BF00656995"> erratum</a>)

and T. Fukushima,

<a href="http://dx.doi.org/10.1007/s001900050271"> Fast transform from

geocentric to geodetic coordinates</a>, J. Geodesy 73, 603&ndash;610 (2003).

However the choice of independent variables in these methods leads

to a loss of accuracy for points near the equatorial plane.

<center>

Back to \ref geodesic.  Forward to \ref geoid.  Up to \ref contents.

</center>

**********************************************************************/

/**

\page geoid Geoid height

<center>

Back to \ref geocentric.  Forward to \ref utilities.  Up to \ref contents.

</center>

The gravitational equipotential surface approximately coinciding with

mean sea level is called the geoid.  The GeographicLib::Geoid class

evaluates the height of the geoid above the WGS84 ellipsoid.  This can

be used to convert heights above mean sea level to heights above the

WGS84 ellipoid.  Because the normal to the ellipsoid differs from the

normal to the geoid (the direction of a plumb line) there is a slight

ambiguity in the measurement of heights; however for heights up to 10km

this ambiguity is only 1mm.

The geoid is usually approximated by an "earth gravity model".  The

models published by the NGA are:

- EGM84:

http://earth-info.nga.mil/GandG/wgs84/gravitymod/wgs84_180/wgs84_180.html

- EGM96:

http://earth-info.nga.mil/GandG/wgs84/gravitymod/egm96/egm96.html

- EGM2008:

http://earth-info.nga.mil/GandG/wgs84/gravitymod/egm2008

.

GeographicLib::Geoid offers a uniform way to handle all 3 geoids at a

variety of grid resolutions.  (In contrast, the software tools that NGA

offers are different for each geoid, and the interpolation programs are

different for each grid resolution.  In addition these tools are written

in Fortran with is nowadays difficult to integrate with other software.)

Unlike other components of %GeographicLib, there is a appreciable error

in the results obtained (at best, the RMS error is 1mm).  However the

class provides methods to report the maximum and RMS errors in the

results.

The class also returns the gradient of the geoid.  This can be used to

estimate the direction of a plumb line relative to the WGS84 ellipsoid.

\section geoidinst Installing the datasets

The geoid heights are computed using interpolation into a rectangular

grid.  The grids are read from data files which have been are computed

using the NGA synthesis programs in the case of the EGM84 and EGM96

models and using the NGA binary gridded data files in the case of

EGM2008.  These data files are available for download:

- EGM84,    30' grid:

<a href="http://sourceforge.net/projects/geographiclib/files/geoids-distrib/egm84-30.tar.bz2/download">tar.bz2</a>,

<a href="http://sourceforge.net/projects/geographiclib/files/geoids-distrib/egm84-30.zip/download">zip</a>.

- EGM84,    15' grid:

<a href="http://sourceforge.net/projects/geographiclib/files/geoids-distrib/egm84-15.tar.bz2/download">tar.bz2</a>,

<a href="http://sourceforge.net/projects/geographiclib/files/geoids-distrib/egm84-15.zip/download">zip</a>.

- EGM96,    15' grid:

<a href="http://sourceforge.net/projects/geographiclib/files/geoids-distrib/egm96-15.tar.bz2/download">tar.bz2</a>,

<a href="http://sourceforge.net/projects/geographiclib/files/geoids-distrib/egm96-15.zip/download">zip</a>.

- EGM96,     5' grid:

<a href="http://sourceforge.net/projects/geographiclib/files/geoids-distrib/egm96-5.tar.bz2/download">tar.bz2</a>,

<a href="http://sourceforge.net/projects/geographiclib/files/geoids-distrib/egm96-5.zip/download">zip</a>.

- EGM2008,   5' grid:

<a href="http://sourceforge.net/projects/geographiclib/files/geoids-distrib/egm2008-5.tar.bz2/download">tar.bz2</a>,

<a href="http://sourceforge.net/projects/geographiclib/files/geoids-distrib/egm2008-5.zip/download">zip</a>.

- EGM2008, 2.5' grid:

<a href="http://sourceforge.net/projects/geographiclib/files/geoids-distrib/egm2008-2_5.tar.bz2/download">tar.bz2</a>,

<a href="http://sourceforge.net/projects/geographiclib/files/geoids-distrib/egm2008-2_5.zip/download">zip</a>.

- EGM2008,   1' grid:

<a href="http://sourceforge.net/projects/geographiclib/files/geoids-distrib/egm2008-1.tar.bz2/download">tar.bz2</a>,

<a href="http://sourceforge.net/projects/geographiclib/files/geoids-distrib/egm2008-1.zip/download">zip</a>.

Download \e either the tar.bz2 file \e or zip file (they have the same

contents).  If possible use the tar.bz2 format, since bzip2 compresses

these files about 2 times better than zip.  The data in tar and zip

files both unpack into a "geoids" directory and, for convenience, all

the data should be unpacked in the same parent directory.  For example

\verbatim

mkdir -p /usr/local/share/GeographicLib

tar xfjC egm96-5.tar.bz2 /usr/local/share/GeographicLib

tar xfjC egm2008-2_5.tar.bz2 /usr/local/share/GeographicLib

etc.

\endverbatim

or

\verbatim

mkdir -p /usr/local/share/GeographicLib

cd /usr/local/share/GeographicLib

unzip ~-/egm96-5.zip

unzip ~-/egm2008-2_5.zip

etc.

\endverbatim

Geoid uses a compile time default (specified by the GEOID_DEFAULT_PATH

macro) to locate the datasets.  This is

- /usr/local/share/GeographicLib/geoids, for non-Windows systems

- C:/cygwin/usr/local/share/GeographicLib/geoids, for Windows systems

.

This may be overridden at run-time by defining the GEOID_PATH

environment variable.  Finally, the path may be set using the optional

second argument to the GeographicLib::Geoid::Geoid constructor.

The sizes of these datasets are

\verbatim

name         geoid    grid           sizes (MB)

tar.bz2    zip    disk

egm84-30     EGM84    30'      0.5       0.5     0.5

egm84-15     EGM84    15'      1.5       1.9     2.0

egm96-15     EGM96    15'      1.5       1.9     2.0

egm96-5      EGM96     5'      9.8        16      18

egm2008-5    EGM2008   5'       10        16      18

egm2008-2_5  EGM2008   2.5'     34        62      72

egm2008-1    EGM2008   1'      155       372     445

\endverbatim

The final "disk" column is the size of the uncompressed data and it also

gives the memory requirements for caching the entire dataset using the

GeographicLib::Geoid::CacheAll method.

\section geoidformat The format of the geoid data files

The gridded data used by the GeographicLib::Geoid class is stored in

16-bit PGM files.  Thus the data for egm96-5 might be stored in the file

- /usr/local/share/GeographicLib/geoids/egm96-5.pgm

.

PGM simple graphic format with the following properties

- it is well documented

<a href="http://netpbm.sourceforge.net/doc/pgm.html">here</a>;

- there are no separate "little-endian" and "big-endian" versions;

- it uses 1 or 2 bytes per pixel;

- pixels can be randomly accessed;

- comments can be added to the file header;

- it is sufficiently simple that it can be easily read without using the

<a href="http://netpbm.sourceforge.net/doc/libnetpbm.html">libnetpbm</a>

library (and thus we avoid adding a software dependency to

%GeographicLib).

.

The major drawback of this format is that since there are only 65535

possible pixel values, the height must be quantized to 3mm.  However,

the resulting quantization error (up to 1.5mm) is typically smaller than

the linear interpolation errors.  The comments in the header for

egm96-5 are

\verbatim

# Geoid file in PGM format for the GeographicLib::Geoid class

# Description WGS84 EGM96, 5-minute grid

# URL http://earth-info.nga.mil/GandG/wgs84/gravitymod/egm96/egm96.html

# DateTime 2009-08-29 18:45:03

# MaxBilinearError 0.140

# RMSBilinearError 0.005

# MaxCubicError 0.003

# RMSCubicError 0.001

# Offset -108

# Scale 0.003

# Origin 90N 0E

# AREA_OR_POINT Point

# Vertical_Datum WGS84

\endverbatim

Of these lines, the Scale and Offset lines are required and define the

conversion from pixel value to height (in meters) using \e height =

\e offset + \e scale \e pixel.  The Geoid constructor also reads the

Description, DataTime, and error lines (if present) and stores the

resulting data so that it can be returned by

GeographicLib::Geoid::Description, GeographicLib::Geoid::DateTime,

GeographicLib::Geoid::MaxError, and GeographicLib::Geoid::RMSError

methods.  The other lines serve as additional documentation but are not

used by this class.  Accompanying egm96-5.pgm (and similarly with the

other geoid data files) are two files egm96-5.wld and

egm96-5.pgm.aux.xml.  The first is an ESRI "world" file and the second

supplies complete projection metadata for use by

<a href="http://www.gdal.org">GDAL</a>.  Neither of these files is read

by GeographicLib::Geoid.

You can use gdal_translate to convert the data files to a standard

GeoTiff, e.g., with

\verbatim

gdal_translate -ot Float32 -scale 0 65000 -108 87 egm96-5.pgm egm96-5.tif

\endverbatim

The arguments to -scale here are specific to the Offset and Scale

parameters used in the pgm file (note 65000 * 0.003 - 108 = 87).  You

can check these by running \ref geoideval with the "-v" option.

Finally, here is a sample script which uses GDAL to create a 1-degree

squared grid of geoid heights at 3&quot; resolution (matching DTED1) by

bilinear interpolation.

\verbatim

#! /bin/sh

lat=37

lon=067

res=3                           # resolution in seconds

TEMP=`mktemp junkXXXXXXXXXX`    # temporary file for GDAL

gdalwarp -q -te `echo $lon $lat $res | awk '{

lon = $1; lat = $2; res = $3;

printf "%.14f %.14f %.14f %.14f",

lon  -0.5*res/3600, lat  -0.5*res/3600,

lon+1+0.5*res/3600, lat+1+0.5*res/3600;

}'` -ts $((3600/res+1)) $((3600/res+1)) -r bilinear egm96-5.tif $TEMP

gdal_translate -quiet \

-mo AREA_OR_POINT=Point \

-mo Description="WGS84 EGM96, $res-second grid" \

-mo Vertical_Datum=WGS84 \

-mo Tie_Point_Location=pixel_corner \

$TEMP e$lon-n$lat.tif

rm -f $TEMP

\endverbatim

\section geoidinterp Interpolating the geoid data

GeographicLib::Geoid evaluates the geoid height is by bilinear or cubic

interpolation.  The gradient of the geoid height is obtained by

differentiating the interpolated height and referencing the result to

distance on the WGS84 ellipsoid.

The bilinear interpolation is based on the values at the 4 corners of

the enclosing cell.  The interpolated height is a continuous function of

position; however the gradient has discontinuities are cell boundaries.

The quantization of the data files exacerbates the errors in the

gradients.

The cubic interpolation is a least-squares fit to the values on a

12-point stencil with weights as follows:

\verbatim

. 1 1 .

1 2 2 1

1 2 2 1

. 1 1 .

\endverbatim

The cubic is constrained to be independent of longitude when evaluating

the height at one of the poles.  Cubic interpolation is considerably

more accurate than bilinear interpolation; however, in this

implementation there are small discontinuities in the heights are cell

boundaries.  The over-constrained cubic fit slightly reduces the

quantization errors on average.

The algorithm for the least squares fit is taken from, F. H. Lesh,

Multi-dimensional least-squares polynomial curve fitting, CACM 2, 29-30

(1959).  This algorithm is not part of Geographic::Geoid; instead it is

implemented as

<a href="http://en.wikipedia.org/wiki/Maxima_(software)">Maxima</a>

code which is used to precompute the matrices to convert the function

values on the stencil into the coefficients from the cubic polynomial.

This code is included as a comment in Geoid.cpp.

The interpolations methods are quick and give good accuracy.  Here is a

summary of the combined quantization and interpolation errors for the

heights.

\verbatim

bilinear error    cubic error

name         geoid    grid     max     rms       max     rms

egm84-30     EGM84    30'      1.546m  70mm      0.274m  14mm

egm84-15     EGM84    15'      0.413m  18mm      0.020m   1mm

egm96-15     EGM96    15'      1.152m  40mm      0.169m   7mm

egm96-5      EGM96     5'      0.140m   5mm      0.003m   1mm

egm2008-5    EGM2008   5'      0.478m  12mm      0.294m   5mm

egm2008-2_5  EGM2008   2.5'    0.135m   3mm      0.031m   1mm

egm2008-1    EGM2008   1'      0.025m   1mm      0.003m   1mm

\endverbatim

The errors are with respect the the specific NGA earth gravity model

(not to any "real" geoid).  The RMS values are obtained using 5 million

uniformly distributed random points.  The maximum values are obtained by

evaluating the errors using a different grid with points at or near the

centers of the original grid.  (The RMS difference between EGM96 and

EGM2008 is about 0.5m.  The RMS difference between EGM84 and EGM96 is

about 1.5m.)

\section geoidcache Caching the geoid data

A simple way of calling Geoid is as follows

\verbatim

#include "GeographicLib/Geoid.hpp"

#include <iostream>

...

GeographicLib::Geoid g("egm96-5");

double lat, lon;

while (std::cin >> lat >> lon)

std::cout << g(lat, lon) << "\n";

...

\endverbatim

The first call to g(lat, lon) causes the data for the stencil points (4

points for bilinear interpolation and 12 for cubic interpolation) to be

read and the interpolated value returned.  A simple 0th-order caching

scheme is provided by default, so that, if the second and subsequent

points falls within the same grid cell, the data values are not reread

from the file.  This is adequate for most interactive use and also

minimizes disk accesses for the case when a continuous track is being

followed.

If a large quantity of geoid calculations are needed, the calculation

can be speeded up by preloading the data for a rectangular block with

GeographicLib::Geoid::CacheArea or the entire dataset with

GeographicLib::Geoid::CacheAll.  If the requested points lie within the

cached area, the cached data values are used; otherwise the data is read

from disk as before.  Caching all the data is a reasonable choice for

the 5' grids and coarser.  Caching all the data for the 1' grid will

require 0.5GB of RAM and should only be used on systems with sufficient

memory.

The use of caching does not affect the values returned.  Because

of the caching and the random file access, this class is \e not

thread safe; i.e., a single instantiation cannot be safely used

by multiple threads.  If multiple threads need to calculate

geoid heights they should all construct thread-local

instantiations.

\section testgeoid Test data for geoids

A test set for the geoid models is available at

- <a href="http://sourceforge.net/projects/geographiclib/files/testdata/GeoidHeights.dat.gz/download">

GeoidHeight.dat.gz</a>

.

This is about 10 MB (compressed).  This test set consists of a set of

500000 geographic coordinates together with the corresponding geoid

heights according to various earth gravity models.

Each line of the test set gives 6 space delimited numbers

- latitude (degrees, exact)

- longitude (degrees, exact)

- EGM84 height (meters, accurate to 1 mm)

- EGM96 height (meters, accurate to 0.1 mm)

- EGM2008 height (meters, accurate to 1 mm)

.

The latitude and longitude are all multiples of 10<sup>-6</sup> deg and

should be regarded as exact.  The geoid heights are computed using the

harmonic NGA synthesis programs.  In the case of the EGM84 and EGM96,

the programs were compiled (with gfortran) and run under Linux.  In the

case of EGM2008, the prescription results in NaNs (possibly because of

uninitialized variables in the program?), so the data was generated with

the precompiled binary that NGA supplies for Windows.

<center>

Back to \ref geocentric.  Forward to \ref utilities.  Up to \ref contents.

</center>

**********************************************************************/

/**

\page utilities Utility Programs

<center>

Back to \ref geocentric.  Forward to \ref old.  Up to \ref contents.

</center>

The examples of utility use here assume that the utilities have been

built and put in a directory in your PATH.

\section geoconvert GeoConvert

GeoConvert is a command line utility for geographic coordinate

conversions via GeographicLib::GeoCoords.  There is an online interface

to this utility at

http://geographiclib.sourceforge.net/cgi-bin/GeoConvert

Examples:

- echo 33.3 44.4 | GeoConvert -u\n=> 38N 444141 368470

- echo 33.3 44.4 | GeoConvert -m -p -3\n=> 38SMB4484

Here is the usage (obtained from "GeoConvert -h")

\verbatim

Usage: GeoConvert [-g|-d|-u|-m|-c] [-p prec] [-z zone] [-s] [-n] [-h]

Convert geographic coordinates to

-g latitude and longitude (decimal degrees), default output

-d latitude and longitude (degrees mins secs)

-u UTM or UPS

-m MGRS

-c meridian convergence and scale

The WGS84 model of the earth is used.  Geographic coordinates are given on

standard input as:

Latitude and longitude (decimal degrees or degrees minutes seconds).  d,

', and " are used to denote degrees, minutes, and seconds, with the least

significant designator optional.  Latitude is given first unless a

hemisphere is specified, e.g., the following are all equivalent

33.3 44.4

E44.4 N33.3

33d18'N 44d24'E

44d24 33d18N

UTM or UPS given as zone+hemisphere easting northing or easting northing

zone+hemisphere.  The zone is absent for a UPS specification.  E.g.,

38N 444140.54 3684706.36

444140.54 3684706.36 38N

S 2173854.98 2985980.58

2173854.98 2985980.58 S

MRGS is used to specify the center of a grid square, e.g.,

38SMB4484

38SMB44140847064

-p prec (default 0) sets the precision relative to 1m.  This gives the

number of digits after the decimal point for UTM/UPS.  The number of digits

per coordinate for MGRS is 5 + prec.  For decimal degrees, the number of

digits after the decimal point is 5 + prec.  For DMS (degree, minute,

seconds) output, the number of digits after the decimal point in the

seconds components is 1 + prec; if this is negative then use minutes (prec

= -2 or -3) or degrees (prec <= -4) as the least significant component.

Print convergence, resp. scale, with 5 + prec, resp. 7 + prec, digits after

the decimal point.  The minimum value of prec is -5 and the maximum is 9

for UTM/UPS, 9 for decimal degrees, 10 for DMS, 6 for MGRS, and 8 for

convergence and scale.

MGRS coordinates are given by truncating (instead of rounding) the

coordinates to the requested precision.  For example is prec = -3, the

result is the 1km square enclosing the position, for example,

echo 38N 444800 3684700 | GeoConvert -m -p -3   ==> 38SMB4484

If the -n option is given, then, on input, an MSGS coordinate refers to

the south-west corner of the MGRS square instead of the center.  Thus:

echo 38SMB4484 | GeoConvert -u         ==> 38N 444500 3684500

echo 38SMB4484 | GeoConvert -u -n      ==> 38N 444000 3684000

Convergence is the bearing of grid north given as degrees clockwise from

true north.

UTM/UPS and MGRS are given in zone of the input if applicable, otherwise in

the standard zone.

-z zone sets the zone for output.  Use either a positive number for a UTM

zone or zone = 0 to specify UPS.  Alternatively use a zone+hemisphere

designation (and the hemisphere is ignored).

-s uses the standard zone.

For example, the point

79.9S 6.1E

corresponds to possible MGRS coordinates

32CMS4324728161 (standard UTM zone = 32)

31CEM6066227959 (neighboring UTM zone = 31)

BBZ1945517770 (neighboring UPS zone)

then

echo 79.9S 6.1E      | GeoConvert -p -3 -m       ==> 32CMS4328

echo 31CEM6066227959 | GeoConvert -p -3 -m       ==> 31CEM6027

echo 31CEM6066227959 | GeoConvert -p -3 -m -s    ==> 32CMS4328

echo 31CEM6066227959 | GeoConvert -p -3 -m -z 0  ==>   BBZ1917

-h prints this help.

\endverbatim

\section geod Geod

Geod is a command line utility for geodesic calculations via

GeographicLib::Geodesic.  There is an online interface to this utility

at http://geographiclib.sourceforge.net/cgi-bin/Geod

Example (route from JFK Airport to Singapore Changi Airport)

- echo "40d38'23N" "073d46'44W" "01d21'33N" "103d59'22E" |\n

Geod -i -d -p 0\n

=> 003d18'29.9" 177d29'09.2" 15347628 4302458\n

This returns the azimuths at the two end points, the distance, and

the "reduced length".  The reduced length gives a measure of the

sensitivity of the end point to errors in the initial azimuth.

- for ((i = 0; i <= 16; i += 2)); do echo ${i}000000;done |\n

Geod -l "40d38'23N" "073d46'44W" "003d18'29.9" -d -p 0\n

=> way points at intervals of 2000 km on route\n

40d38'23.0&quot;N 073d46'44.0&quot;W 003d18'29.9&quot; 0\n

58d34'45.1&quot;N 071d49'36.7&quot;W 004d48'48.8&quot; 1967419\n

76d22'28.4&quot;N 065d32'17.8&quot;W 010d41'38.4&quot; 3743642\n

84d50'28.0&quot;N 075d04'39.2&quot;E 150d55'00.9&quot; 5156905\n

67d26'20.3&quot;N 098d00'51.2&quot;E 173d27'20.3&quot; 6070415\n

49d33'03.2&quot;N 101d06'52.6&quot;E 176d07'54.3&quot; 6394568\n

31d34'16.5&quot;N 102d30'46.3&quot;E 177d03'08.4&quot; 6095725\n

13d31'56.0&quot;N 103d26'50.7&quot;E 177d24'55.0&quot; 5200700\n

04d32'05.7&quot;S 104d14'48.7&quot;E 177d28'43.6&quot; 3795596

Here is the usage (obtained from "Geod -h")

\verbatim

Usage: Geod [-l lat1 lon1 azi1 | -i] [-a] [-n | -e a r]

[-d] [-b] [-f] [-p prec] [-h]

Perform geodesic calculations.

The shortest path between two points on the ellipsoid at (lat1, lon1) and

(lat2, lon2) is called the geodesic.  Its length is s12 and the geodesic

from point 1 to point 2 has azimuths azi1 and azi2 at the two end

points.  The reduced length of the geodesic, m12, is defined such that

if the initial azimuth is perturbed by dazi1 (radians) then the second

point is displaced by m12*dazi1 in the direction perpendicular to the

geodesic.  On a flat surface, we have m12 = s12.

Geod operates in one of three modes:

(1) It accepts lines on the standard input containing "lat1 lon1 azi1

s12" and prints "lat2 lon2 azi2 m12" on standard output.  This is

the direct geodesic calculation.

(2) Command line arguments "-l lat1 lon1 azi1" specify a geodesic line.

Geod then accepts a sequence of s12 values (one per line) on

standard input and prints "lat2 lon2 azi2 m12" for each.  This

generates a sequence of points on a single geodesic.

(3) With the -i command line argument, Geod performs the inverse

geodesic calculation.  It reads lines containing "lat1 lon1 lat2

lon2" and prints the corresponding values of "azi1 azi2 s12 m12".

By default, the WGS84 ellipsoid is used.  Specifying "-e a r" sets the

equatorial radius of the ellipsoid to "a" and the reciprocal flattening

to r.  Setting r = 0 results in a sphere.  Specify r < 0 for a prolate

ellipsoid.  The -n option uses the international ellipsoid (equivalent to

"-e 6378388 297").

Output of angles is as decimal degrees.  If -d is specified the output

is as degrees, minutes, seconds.  Input can be in either style.  d, ',

and " are used to denote degrees, minutes, and seconds, with the least

significant designator optional.  By default, latitude precedes

longitude for each point; however on input either may be given first by

appending N or S to the latitude and E or W to the longitude.  Azimuths

(measured clockwise from north) give the heading of the geodesic.  The

azimuth azi2 is the forward azimuth (the heading beyond point 2).  If

the -b flag is given, azi2 is converted to a back azimuth (the direction

back to point 1) for output.

s12 is given in meters, unless the -a flag is given.  In that case, s12

(on both input and output) are given as the arc length on the auxiliary

sphere a12 (measured in degrees).  In these terms, 180 degrees is the

distance from one equator crossing to the next or from the minimum

latitude to the maximum latitude.  Distances greater than 180 degrees do

not correspond to shortest paths.  m12 is always given in meters.

The output lines consist of the four quantities needed to complete the

specification of the geodesic.  With the -f option, each line of output

is a complete geodesic specification consisting of nine quantities

lat1 lon1 azi1 lat2 lon2 azi2 s12 a12 m12

where here s12 is the distance and a12 the arc length.

-p prec (default 3) gives the precision of the output relative to 1m.

The minimum value of prec is 0 (1 m accuracy) and the maximum value is

10 (0.1 nm accuracy, but then the last digits are unreliable).

-h prints this help.

\endverbatim

\section transversemercatortest TransverseMercatorTest

TransverseMercatorTest is a command line utility for testing

GeographicLib::TransverseMercatorExact and

GeographicLib::TransverseMercator.

Examples:

- echo 0 90 | TransverseMercatorTest\n=>

0 90 25953592.84 9997964.94 90 18.40

- echo 260e5 100e5 | TransverseMercatorTest -r\n=>

-0.02 90.00 26000000 10000000 90.01 18.47

Note that, unlike \ref geoconvert, TransverseMercatorTest has no

no ability to change the precision of the output.

Here is the usage (obtained from "TranverseMercatorTest -h")

\verbatim

TransverseMercatorTest [-r] [-t|-s]

Convert between geographic coordinates and transverse Mercator coordinates.

Read lines with latitude and longitude (or easting and northing if -r is

specified) from standard input and print latitude, longitude, easting,

northing, convergence, and scale.  Units are degrees (or DMS) and meters.

By default, the WGS84 is ellipsoid is used, central meridian = 0, UTM

central scale (0.9996), and false easting and false northing are zero.

If -r is given, the reverse projection is performed (the inputs are easting

and northing).

If -s is given, the sixth-order Krueger series approximation to the

transverse Mercator projection is used instead of the exact projection.

If -t is specified, an ellipsoid of eccentricity 0.1 is used, central scale

= 1, 1/4 meridian distance = 1.  In addition, the cut in the exact

transverse Mercator projection at northing = 0 is removed.  The domain of

latitude (lat) and longitude (lon) is the union of

lat in [0, 90]  and lon in [0, 90]

lat in (-90, 0] and lon in [81, 90]

The domain of easting (x) and northing (x) is the union of

x in [0, inf)       and y in [0, 1]

x in [1.71..., inf) and y in (-inf, 0]

-s and -t are mutually exclusive (the last flag specified is the operative

one).

-h prints this help.

\endverbatim

\section cartconvert CartConvert

CartConvert is a command line utility for geodetic to cartesian

coordinate conversions via GeographicLib::Geocentric and

GeographicLib::LocalCartesian.

Examples:

- echo 33.3 44.4 6000 | CartConvert\n=>

3816209.60 3737108.55 3485109.57

- echo 33.3 44.4 6000 | CartConvert -l 33 44 20\n=>

37288.97 33374.29 5783.64

- echo 30000 30000 0 | CartConvert -r\n=>

6.483 45 -6335709.73

Note that, unlike \ref geoconvert, CartConvert has no

no ability to change the precision of the output.

Here is the usage (obtained from "CartConvert -h")

\verbatim

Usage: CartConvert [-r] [-l lat0 lon0 h0] [-h]

Convert geodetic coordinates to either geocentric or local cartesian

coordinates.  Geocentric coordinates have the origin at the center of the

earth, with the z axis going thru the north pole, and the x axis thru lat =

0, lon = 0.  By default, the conversion is to geocentric coordinates.

Specifying -l lat0 lon0 h0 causes a local coordinate system to be used with

the origin at latitude = lat0, longitude = lon0, height = h0, z normal to

the ellipsoid and y due north.

By default, the WGS84 ellipsoid is used.  Specifying "-e a r" sets the

equatorial radius of the ellipsoid to "a" and the reciprocal flattening

to r.  Setting r = 0 results in a sphere.  Specify r < 0 for a prolate

ellipsoid.

Geodetic coordinates are provided on standard input as a set of lines

containing (blank separated) latitude, longitude (degrees or DMS), and

height (meters).  For each set of geodetic coordinates, the corresponding

cartesian coordinates x, y, z (meters) are printed on standard output.

If -r is given the reverse transformation is performed.

-h prints this help

\endverbatim

\section equidistanttest EquidistantTest

EquidistantTest is a command line utility for performing azimuthal

equidistant and Cassini-Soldner projections using

GeographicLib::AzimuthalEquidistant and GeographicLib::CassiniSoldner.

These projections are both defined in terms of geodesics.

Examples (position of Caen in the Cassini-Soldner projection with origin

at the Paris Observatory)

- echo 48.648 -2.007 | EquidistantTest -c 48.836 2.337\n=>

-319919 -11791 86.7 0.999

- echo -319919 -11791 | EquidistantTest -c 48.836 2.337 -r\n=>

48.648 -2.007 86.7 0.999

Note that, unlike \ref geoconvert, EquidistantTest has no

no ability to change the precision of the output.

Here is the usage (obtained from "EquidistantTest -h")

\verbatim

Usage: EquidistantTest [-c lat0 lon0] [-z lat0 lon0] [-r] [-h]

Convert geodetic coordinates to either azimuthal equidistant or

Cassini-Soldner coordinates.  The center of the projection (lat0, lon0)

is specified by either the -c option (for Cassini-Soldner) or -z option

(for azimuthal equidistant).  At least one of these options must be

given (the last one given is used).  The WGS84 model of the earth is

used.

Geodetic coordinates are provided on standard input as a set of lines

containing (blank separated) latitude and longitude (degrees or DMS).

For each set of geodetic coordinates, the corresponding projected

coordinates x, y (meters) are printed on standard output together with

the azimuth azi (degrees) and reciprocal scale rk.  For Cassini-Soldner,

azi is the bearing of the easting direction and the scale in the

northing direction is 1/rk.  For azimuthal equidistant, azi is the

bearing of the radial direction and the scale in the azimuthal direction

is 1/rk

If -r is given the reverse transformation is performed.  x and y are

given on standard input and each line of the standard output gives

latitude, longitude, azi, and rk.

-h prints this help

\endverbatim

\section geoideval GeoidEval

Evaluate the height of the geoid by interpolating into a grid via

GeographicLib::Geoid.  There is an online interface to this utility

at http://geographiclib.sourceforge.net/cgi-bin/Geoid

Example: the height of the EGM96 geoid at Timbuktu

- echo "16d46'33N" "3d00'34W" | GeoidEval\n=>

28.7068 -0.02e-6 -1.73e-6

.

The first number is the height of the geoid and the 2nd and 3rd are its

slopes in the northerly and easterly direction.

In addition, this utility requires the installation of the geoid data

files; see \ref geoidinst for details

Here is the usage (obtained from "GeoidEval -h")

\verbatim

Usage:

GeoidEval [-n name] [-d dir] [-l] [-a] [-c south west north east] [-v] [-h]

Read in positions on standard input and print out the corresponding

geoid heights on standard output.  In addition print the northly and

easterly gradients of the geoid height (i.e., the rate at which the

geoid height changes per unit distance along the WGS84 ellipsoid in

the specified directions).

Positions are given as latitude and longitude, UTM/UPS, or MGRS, in

any of the formats accepted by GeoConvert.

By default the EGM96 geoid is used with a 5' grid.  This may be

overriden with the -n option.  The name specified should be one of

bilinear error    cubic error

name         geoid    grid     max     rms       max     rms

egm84-30     EGM84    30'      1.546m  70mm      0.274m  14mm

egm84-15     EGM84    15'      0.413m  18mm      0.020m   1mm

egm96-15     EGM96    15'      1.152m  40mm      0.169m   7mm

egm96-5      EGM96     5'      0.140m   5mm      0.003m   1mm

egm2008-5    EGM2008   5'      0.478m  12mm      0.294m   5mm

egm2008-2_5  EGM2008   2.5'    0.135m   3mm      0.031m   1mm

egm2008-1    EGM2008   1'      0.025m   1mm      0.003m   1mm

(Some of the geoids may not be available.)  The errors listed here

are estimates of the quantization and interpolation errors in the

reported heights compared to the specified geoid.

Cubic interpolation is used to compute the geoid height unless

-l is specified in which case bilinear interpolation is used.

Cubic interpolation is more accurate; however it results in

small discontinuities in the returned height on cell boundaries.

The gradients are computed by differentiating the interpolated

results.

GeoidEval will load the geoid data from the directory specified by

the -d option.  If this is not provided, it will look up the value of

GEOID_PATH (currently UNDEFINED) in the environment.

If this is not defined, it will use the compile-time value of

/usr/local/share/GeographicLib/geoids.

By default, the data file is randomly read to compute the geoid

heights at the input positions.  Usually this is sufficient for

interactive use.  If many heights are to be computed, GeoidEval

allows a block of data to be read into memory and heights within the

corresponding rectangle can then be computed without any disk acces.

If -a is specified all the geoid data is read; in the case of

egm2008-1, this requires about 0.5 GB of RAM.  The -c option allows

a rectangle of data to be cached.  The evaluation of heights

outside the cached rectangle causes the necessary data to be read

from disk.

Regardless of whether any cache is requested (with the -a or -c

options), the data for the last grid cell in cached.  This allows

the geoid height along a continuous path to be returned with little

disk overhead.

The -v option causes the data about the current geoid to be printed

to standard error.

-h prints this help.

\endverbatim

<center>

Back to \ref geocentric.  Forward to \ref old.  Up to \ref contents.

</center>

**********************************************************************/

/**

\page old Old Versions

<center>

Back to \ref utilities.  Up to \ref contents.

</center>

List of versions in reverse chronological order together with a brief

list of changes.  (Note: Old versions of the library use a year-month

style of numbering.  Now, the library uses

<a href="http://www.gnu.org/software/libtool/manual/libtool.html#Versioning">

libtool's version numbering scheme</a> using an interface and revision

numbers.)

- <a href="http://sourceforge.net/projects/geographiclib/files/distrib/geographiclib-1.2.tar.gz/download">1.2 (released 2010-05-21)</a>

- FIX BUGS in GeographicLib::Geodesic,

- wrong azimuth returned by Direct if point 2 is on a pole;

- Inverse sometimes fails with very close points.

- Improve calculation of scale in GeographicLib::CassiniSoldner,

- add GeographicLib::GeodesicLine::Scale,

GeographicLib::GeodesicLine::EquatorialAzimuth, and

GeographicLib::GeodesicLine::EquatorialArc;

- break friend connection between CassiniSoldner and Geodesic.

- Add GeographicLib::DMS::DecodeAngle and

GeographicLib::DMS::DecodeAzimuth.  Extend

GeographicLib::DMS::Decode and GeographicLib::DMS::Encode to deal

with distances.

- Code and documentation changes in GeographicLib::Geodesic and

GeographicLib::Geocentric for consistency with

the forthcoming paper on geodesics.

- Increase order of series using in GeographicLib::Geodesic to 6 (full

accuracy maintained for ellipsoid flattening < 0.01).

- Macro __NO_LONG_DOUBLE_MATH to disable use of long double.

- Correct declaration of GeographicLib::Math::isfinite to return a bool.

- Changes in the \ref utilities,

- improve error reporting when parsing command line arguments;

- accept latitudes and longitudes in decimal degrees or degrees,

minutes, and seconds, with optional hemisphere designators;

- \ref geoconvert -z accepts zone or zone+hemisphere;

- \ref geoideval accepts any of the input formats used by \ref geoconvert;

- \ref cartconvert allows the ellipsoid to be specified with -e.

- <a  href="http://sourceforge.net/projects/geographiclib/files/distrib/geographiclib-1.1.tar.gz/download">1.1 (released 2010-02-09)</a>

- FIX BUG (introduced in 2009-03) in EllipticFunction::E(sn,cn,dn).

- Increase accuracy of scale calculation in TransverseMercator and

TransverseMercatorExact.

- Code and documentation changes for consistency with

<a  href="http://arxiv.org/abs/1002.1417">arXiv:1002.1417</a>

- <a  href="http://sourceforge.net/projects/geographiclib/files/distrib/geographiclib-1.0.tar.gz/download">

1.0 (released 2010-01-07)</a>

- Add autoconf configuration files.

- BUG FIX: Improve initial guess for Newton's method in

PolarStereographic::Reverse.  (Previously this failed to converge

when the co-latitude exceeded about 130 deg.)

- Constructors for TransverseMercator, TransverseMercatorExact,

PolarStereographic, Geocentric, and Geodesic now check for obvious

problems with their arguments and throw an exception if necessary.

- Most classes now include inspector functions such as MajorRadius()

so that you can determine how instances were constructed.

- Add GeographicLib::LambertConformalConic class.

- Add GeographicLib::PolarStereographic::SetScale to allow the

latitude of true scale to be specified.

- Add solution and project files for Visual Studio 2008.

- Add GeographicLib::GeographicErr for exceptions.

- GeographicLib::Geoid changes:

- BUG FIX: fix typo in GeographicLib::Geoid::Cache which could cause

a segmentation fault in some cases when the cached area spanned

the prime meridian.

- Include sufficient edge data to allow heights to be returned for

cached area without disk reads;

- Add inspector functions to query the extent of the cache.

- <a href="http://sourceforge.net/projects/geographiclib/files/distrib/Geographic-2009-11.tgz/download">2009-11</a>

- Allow specification of "closest UTM zone" in GeographicLib::UTMUPS

and \ref geoconvert (via -t option).

- Utilities now complain is there are too many tokens on input lines.

- Include real-to-real versions of GeographicLib::DMS::Decode and

GeographicLib::DMS::Encode.

- More house-cleaning changes:

- Ensure that functions which return results through reference

arguments do not alter the arguments when an exception is thrown.

- Improve accuracy of GeographicLib::MGRS::Forward.

- Include more information in some error messages.

- Improve accuracy of inverse hypobolic functions.

- Fix the way GeographicLib::Math functions handle different precisions.

- <a href="http://sourceforge.net/projects/geographiclib/files/distrib/Geographic-2009-10.tgz/download">2009-10</a>

- Change web site to http://geographiclib.sourceforge.net

- Several house-cleaning changes:

- Change from the a flat directory structure to a more easily

maintained one.

- Introduce Math class for common mathematical functions (in

Constants.hpp).

- Use Math::real as the type for all real quantities.  By default this

is typedefed to double; and the library should be installed this

way.

- Eliminate const reference members of AzimuthalEquidistant,

CassiniSoldner and LocalCartesian so that they may be copied.

- Make several constructors explicit.  Disallow some constructors.

Disallow copy constructor/assignment for Geoid.

- Document least square formulas in Geoid.cpp.

- Use unsigned long long for files positions of geoid files in Geoid.

- Introduce optional mgrslimits argument in UTMUPS::Forward and

UTMUPS::Reverse to enforce stricter MGRS limits on eastings and

northings.

- Add 64-bit targets in Visual Studio project files.

- <a href="http://sourceforge.net/projects/geographiclib/files/distrib/Geographic-2009-09.tgz/download">2009-09</a>

- Add GeographicLib::Geoid and \ref geoideval utility.

- <a href="http://sourceforge.net/projects/geographiclib/files/distrib/Geographic-2009-08.tgz/download">2009-08</a>

- Add GeographicLib::CassiniSoldner class and \ref equidistanttest utility.

- Fix bug in GeographicLib::Geodesic::Inverse where NaNs were

sometimes returned.

- INCOMPATIBLE CHANGE: AzimuthalEquidistant now returns the reciprocal

of the azimuthal scale instead of the reduced length.

- Add -n option to \ref geoconvert.

- <a href="http://sourceforge.net/projects/geographiclib/files/distrib/Geographic-2009-07.tgz/download">2009-07</a>

- Speed up the series inversion code in tmseries.mac and geod.mac.

- Reference Borkowski in section on \ref geocentric.

- <a href="http://sourceforge.net/projects/geographiclib/files/distrib/Geographic-2009-06.tgz/download">2009-06</a>

- Add routines to decode and encode zone+hemisphere to GeographicLib::UTMUPS.

- Clean up code in GeographicLib::Geodesic.

- <a href="http://sourceforge.net/projects/geographiclib/files/distrib/Geographic-2009-05.tgz/download">2009-05</a>

- Improvements to GeographicLib::Geodesic:

- more economical series expansions,

- return reduced length (as does the \ref geod utility),

- improved calculation of starting point for inverse method,

- use reduced length to give derivative for Newton's method.

- Add GeographicLib::AzimuthalEquidistant class.

- Make GeographicLib::Geocentric, GeographicLib::TransverseMercator,

and GeographicLib::PolarStereographic classes work with prolate

ellipsoids.

- \ref cartconvert checks its inputs more carefully.

- Remove reference to defunct Constants.cpp from GeographicLib.vcproj.

- <a href="http://sourceforge.net/projects/geographiclib/files/distrib/Geographic-2009-04.tgz/download">2009-04</a>

- Use compile-time constants to select the order of series in

GeographicLib::TransverseMercator.

- 2x unroll of Clenshaw summation to avoid data shuffling.

- Simplification of GeographicLib::EllipticFunction::E.

- Use STATIC_ASSERT for compile-time checking of constants.

- Improvements to GeographicLib::Geodesic:

- compile-time option to change order of series used,

- post Maxima code for generating the series,

- tune the order of series for double,

- improvements in the selection of starting points for Newton's

method,

- accept and return spherical arc lengths,

- works with both oblate and prolate ellipsoids,

- add -a, -e, -b options to the \ref geod utility.

- <a href="http://sourceforge.net/projects/geographiclib/files/distrib/Geographic-2009-03.tgz/download">2009-03</a>

- Add GeographicLib::Geodesic and the \ref geod utility.

- Declare when no exceptions are thrown by functions.

- Minor changes to GeographicLib::DMS class.

- Use invf = 0 to mean a sphere in constructors to some classes.

- The makefile creates a library and includes an install target.

- Rename GeographicLib::ECEF to GeographicLib::Geocentric, ECEFConvert

to \ref cartconvert.

- Use inline functions to define constant doubles in Constants.hpp.

- <a href="http://sourceforge.net/projects/geographiclib/files/distrib/Geographic-2009-02.tgz/download">2009-02</a>

- Fix documentation of constructors (flattening -> inverse

flattening).

- Use std versions of math functions.

- Add GeographicLib::ECEF and GeographicLib::LocalCartesian classes

and the ECEFConvert utility.

- Gather the documentation on the utility programs onto one page.

- <a href="http://sourceforge.net/projects/geographiclib/files/distrib/Geographic-2009-01.tgz/download">2009-01</a>

- First proper release of library.

- More robust GeographicLib::TransverseMercatorExact:

- Introduce \e extendp version of constructor,

- Test against extended test data,

- Optimize starting positions for Newton's method,

- Fix behavior near all singularities,

- Fix order dependence in C++ start-up code,

- Improved method of computing scale and convergence.

- Documentation on transverse Mercator projection.

- Add GeographicLib::MGRS, GeographicLib::UTMUPS, etc.

- 2008-09

- Ad hoc posting of information on the transverse Mercator projection.

<center>

Back to \ref utilities.  Up to \ref contents.

</center>

**********************************************************************/