Object Type:    fura2

Description:	Computes fura2 fluorescence in a single difshell.

Author:         E. De Schutter BBF-UIA 1/95

------------------------------------------------------------------------------

ELEMENT PARAMETERS

DataStructure:	fura2_type [in src/concen/conc_struct.h]

Size:           104 bytes

Fields:         F340            F340 fluorescence

F380            F380 fluorescence

ratio           F340/F380

------------------------------------------------------------------------------

SIMULATION PARAMETERS

Function:	FuraRatio [in src/concen/fura2.c]

Classes:        concentration  segment

Actions:        RESET  PROCESS

Messages:       CONCEN  Bfree Bbound vol    (from all fura2 difbuffers)

------------------------------------------------------------------------------

Notes:          Computes the fluorescence signal of the fura-2 calcium

indicator dye by adding together information from a series of

difbuffers (representing the fura2 in shells).

In order to use the fura2 object, create difbuffers to model

the diffusion of the fura-2 dye and Ca buffering for each

difshell that is associated with a compartment.  The Btot, D,

kBf, and kBb fields for the difbuffers should be set to values

that are typical of the ionic environment of the dye.  The

difbuffers are linked to their difshells with the usual

messages.

Then send "CONCEN Bfree Bbound vol" messages to a single fura2

element from each difbuffer that corresponds to the

compartment for which you wish to calculate the fura-2

fluorescence signal.

For a general description of buffered calcium diffusion see:

De Schutter E., and Smolen P., "Calcium dynamics in large

neuronal models", in Methods in neuronal modeling: From ions

to networks (2nd edition), C. Koch and I. Segev editors, pp.

211-250 (1998).  Details of the GENESIS implementation of the

fura2 object are given in Sec. 6.6.4.  Also see Blumenfeld et.

al., Biophys. J. 63: 1146-1164 (1992) for additional

discussion of fura-2 parameters.

Example:

See also:	difshell, difbuffer